# ATEC-PANDA

Type 2188

### Type 2168

54 cm Color TV Set

# Service Manual

#### Contents

- Overview………………………………………………………………………………………2

- Chassis Installation and Parts List………………………………………………………………3

- Cabinet Parts List and Installation……………………………………………………………10

- Signal Flow Diagram…………………………………………………………………………14

- Adjustment Test…………………………………………………………………………………15

  - I2C bus adjustment test…………………………………………………………………15

  - Complete set adjustment test……………………………………………………………18

- Trouble Shooting……………………………………………………………………………20

- Overview of IC Function……………………………………………………………………24

- IC and Voltage of Transistor…………………………………………………………………29

- Resistance to Earth of Some Circuits………………………………………………………32

- Waveform Diagrams…………………………………………………………………………34

Appendix 1  Explanation of Parts No.……………………………………………………………35

Appendix 2  Difference of CRT……………………………………………………………………50

Appendix 3  Circuit diagram………………………………………………………………………51

Appendix 4-1  Fig. PCB touch side…………………………………………………………………52

Appendix 4-2  Wire arrangement on PCB welding side…………………………………………53

1. Overview

This TV set Consist of main chip LA76814 controlled by Sanyo I2C bus and CPU LC863232A.  LA76814 is IC dedicated for NTSC, integrated following circuits inside: amplifying and detecting in medium frequency, AFC, demodulating of sound medium frequency, brightness delay-line, filtering and demodulating of color signal, processing of horizontal small signals. In addition to the +B (B1) voltage of chassis controlled by potentiometer, others are controlled by CPU through the bus.

Functional feature:

1. Frequency Tuning (FS) 181 channels System (North & South America)

2. Receiving bands：AIR/STD/HRC/IRC

3. Color System：NTSC-M

4. Green Power Supply, Low STAND-BY Energy Consumption

5. X-Radiation Protection

6. One AV Input (Rear)

7. On Screen Display (OSD):Three Languages: English / Spanish / Portuguese

8. All Functions Remote Control

9. Easy release of all channels with a single key

10. Closed Caption Decoder with CC1 , CC2 , T1 and T2 decoder capabilities

11. Caption only on Mute optionally selectable

12. Mute

13. Half  Mute  (50% Volume pressinga single key)

14. Without releasing MUTE Volume can be diminished

15. Smart Picture key: SOFT , STANDARD , SHARP and PERSONAL

16. Quick View key

17. Time-related features: (Alarm, Timer On/Off , Sleep) independently selectable

18. LOGO screen at Start Up with showing time selectable in TEST mode

20. Favorite Channels

21. MESSAGE feature

22. ALARM Feature up to 99 minutes 59 seconds

23. Scan

24. Volume Plus feature in all 181 channels independently

25. Power ON by pressing any key in the front panel

26. Blue Back, Noise Reducing, Black Stretch , RG gamma correction, Blue gamma correction

optionally selectable by the user

27. Autosearch

28. Call (Display) key in the Remite Control (RC) showing Channel Status, Alarm Time if

activated ,Sleep Time if activated,  Favorite Channels and Clock if running

2. Installation and Parts List of Inner Works

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number	Carbon-film		number	Carbon-film

resistor	　	            resistor

R105	1/6W 22K	RDB223J-NAARD	R157	1/6W 12K	RDB123J-NAARD

R106	1/6W 22K	RDB223J-NAARD	R160	1/6W 470	RDB471J-NAARD

R108	1/6W 390K	RDB394J-NAARD	R163	1/6W 33K	RDB333J-NAARD

R109	1/6W 8.2K	RDB822J-NAARD	R165	1/6W 100K	RDB104J-NAARD

R110	1/6W 8.2K	RDB822J-NAARD	R166	1/6W 22K	RDB223J5NBAND

R111	1/6W 1.5K	RDB152J-NAARD	R190	1/6W 12K	RDB123J-NAARD

R112	1/6W 8.2K	RDB822J-NAARD	R192	1/6W 39K	RDB393J-NAARD

R113	1/6W 390K	RDB394J-NAARD	R202	1/6W 33K	RDB333J-NAARD

R114	1/6W 1M	RDB105J-NAARD	R203	1/6W 100K	RDB104J-NAARD

R115	1/6W 100	RDB101J-NAARD	R204	1/6W 100K	RDB104J-NAARD

R116	1/6W 4.7K	RDB472J-NAARD	R205	1/6W 100	RDB101J-NAARD

R117	1/6W 10K	RDB103J-NAARD	R206	1/6W 100	RDB101J-NAARD

R118	1/6W 27K	RDB273J-NAARD	R207	1/6W 120	RDB121J-NAARD

R119	1/6W 10K	RDB103J-NAARD	R208	1/6W 1K	RDB102J-NAARD

R120	1/6W 1K	RDB102J-NAARD	R215	1/6W 330	RDB331J-NAARD

R121	1/6W 1K	RDB102J-NAARD	R216	1/6W 270	RDB271J-NAARD

R122	1/6W 1K	RDB102J-NAARD	R226	1/6W 8.2K	RDB822J-NAARD

R123	1/6W 1K	RDB102J-NAARD	R314	1/6W 1K	RDB102J-NAARD

R124	1/6W 1K	RDB102J-NAARD	R331	1/6W 3.3K	RDB332J-NAARD

R125	1/6W 1K	RDB102J-NAARD	R345	1/6W 15K	RDB153J-NAARD

R126	1/6W 1K	RDB102J-NAARD	R346	1/6W 10K	RDB103J5NBAND

R127	1/6W 1K	RDB102J-NAARD	R373	1/6W 22K	RDB223J-NAARD

R128	1/6W 220	RDB221J-NAARD	R402	1/6W 3.3K	RDB332J-NAARD

R129	1/6W 220	RDB221J-NAARD	R405	1/6W 560	RDB561J-NAARD

R134	1/6W 1K	RDB102J-NAARD	R406	1/4W 1	RDC1R0J-NABRD

R135	1/6W 100	RDB101J-NAARD	R407	1/2W 330	RDD331J4NBCND

R137	1/6W 4.7K	RDB472J-NAARD	R409	1/6W 4.7K	RDB472J-NAARD

R138	1/6W 4.7K	RDB472J-NAARD	R410	1/6W 4.7K	RDB472J-NAARD

R139	1/6W 100	RDB101J-NAARD	R411	1/6W 18K	RDB183J-NAARD

R140	1/6W 100	RDB101J-NAARD	R412	1/6W 22K	RDB223J-NAARD

R141	1/4W 560	RDC561J-NABRD	R431	1/2W 1.2K	RDD122J4NBCND

R142	1/6W 100K	RDB104J-NAARD	R432	1/2W 680	RDD681J4NBCND

R143	1/6W 10K	RDB103J-NAARD	R501	1/6W 2K	RDB202J-NAARD

R152	1/6W 1.2K	RDB122J-NAARD	R502	1/6W 5.6K	RDB562J-NAARD

R153	1/6W 1.8K	RDB182J-NAARD	R504	1/6W 1K	RDB102J-NAARD

R154	1/6W 2.7K	RDB272J-NAARD	R504A	1/6W 680	RDB681J6NBAND

R155	1/6W 3.9K	RDB392J-NAARD	R506	1/2W 1K	RDD102J4NBCND

R156	1/6W 6.8K	RDB682J-NAARD	R508	1/2W 10K	RDD103J4NBCND

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number	Carbon-film		number	Carbon-film

resistor	　	            resistor

R509	1/6W 10K	RDB103J-NAARD	R764	1/6W 4.7K	RDB472J-NAARD

R514	1/4W 1K	RDC102J-NABRD	R807	1/6W 100	RDB101J-NAARD

R526	1/6W 220K	RDB224J-NAARD	R812	1/6W 100K	RDB104J-NAARD

R601	1/2W 220K	RDD224J5NBCND	R818B	1/6W 6.8K	RDB682J-NAARD

R605	1/6W 4.7K	RDB472J-NAARD	R902	1/2W 1.8K	RDD182J5NBCND

R606	1/6W 120K	RDB124J-NAARD	R905	1/6W 270	RDB271J2NBAND

R607	1/6W 10	RDB100J-NAARD	R912	1/2W 1.8K	RDD182J4NBCND

R608	1/6W 10	RDB100J-NAARD	R915	1/6W 270	RDB271J2NBAND

R610	1/6W 3K	RDB302J-NAARD	R922	1/2W 1.8K	RDD182J4NBCND

R612	1/6W 1K	RDB102J-NAARD	R925	1/6W 270	RDB271J2NBAND

R613	1/4W 22	RDC220J-NABRD	R981	1/6W 2.7K	RDB272J2NBAND

R615	1/6W 820	RDB821J-NAARD	R982	1/6W 2.7K	RDB272J2NBAND

R639	1/2W 47K	RDD473J4NBCND	R983	1/6W 2.7K	RDB272J2NBAND

R639B	1/2W 51K	RDD513J4NBCND	R985	1/6W 33	RDB330J2NBAND

R640	1/4W 2.4K	RDC242J-NABRD	RVD701	1/6W 1.5K	RDB152J-NAARD

R641	1/6W 220	RDB221J5NBAND	R2001	1/6W 10K	RDB103J-NAARD

R642	1/6W 1K	RDB102J-NAARD	R2002	1/6W 10K	RDB103J-NAARD

R643	1/6W 10K	RDB103J-NAARD	R2003	1/6W 8.2K	RDB822J-NAARD

R644	1/6W 47K	RDB473J-NAARD	R2004	1/6W 10K	RDB103J-NAARD

R645	1/6W 470	RDB471J-NAARD	R3000	1/6W 100	RDB101J-NAARD

R651	1/6W 22K	RDB223J-NAARD	R3001	1/6W 100	RDB101J-NAARD

R701	1/6W 100	RDB101J-NAARD

R702	1/6W 100	RDB101J-NAARD

R703	1/6W 100	RDB101J-NAARD	　	Fuse resistor

R704	1/6W 10K	RDB103J-NAARD	R951	2W 3.9 	RFF3R9J5ACDNF

R705	1/6W 100	RDB101J-NAARD

R706	1/6W 100	RDB101J-NAARD

R708	1/6W 1K	RDB102J-NAARD		Glass grazed

R709	1/6W 33	RDB330J-NAARD	　	resistor

R713	1/6W 560K	RDB564J-NAARD	R617	1/2W 120K	RGDX124K504NZ

R748	1/6W 1.2K	RDB122J-NAARD	R618	1/2W 120K	RGDX124K504NZ

R749	1/6W 1K	RDB102J-NAARD	R619	1/2W 24M	RGDX246K504NZ

R754	1/6W 1K	RDB102J-NAARD

R755	1/6W 22K	RDB223J-NAARD		Metal film

R758	1/6W 12K	RDB123J-NAARD	　	resistor

R759	1/6W 12K	RDB123J-NAARD	R403	1/6W 10K	RJB103F-NAARD

R762	1/6W 1.2K	RDB122J-NAARD	R757	1/6W 4.7K    	RJB472F-NAARD

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number	Metal oxide		number	Ceramic

resistor                                               capacitor

R104	2W 8.2K	RSF822J7ACDND	C109	50V 0.01u  	CKF103Z1FA-FC

R229	2W 56	RSF560J7ACDND	C110	50V 0.01u  	CKF103Z1FA-FC

R404	1W 1.5	RSE1R5J5DCCND	C114	50V 560P	CKF561K1BA-FC

R503	1W 270	RSE271J4ACCND	C115	50V 560P	CKF561K1BA-FC

R507	2W 270	RSF271J7ACDND	C116	50V 560P	CKF561K1BA-FC

R510	1W 1K	RSE102J5ACCND	C117	50V 560P	CKF561K1BA-FC

R525	1W 12K	RSE123J4ACCND	C118	50V 100P	CCF101J1CA-FC

R600	1W 1K	RSE102J4ACCND	C119	50V 0.01u	CKF103Z1FA-FC

R604	3W 47K	RSG473J7ACEND	C124	50V 0.01u	CKF103Z1FA-FC

R604B	3W 47K	RSG473J7ACEND	C127	50V 0.01u	CKF103Z1FA-FC

R630	2W 56	RSF560J7ACDND	C128	50V 220P	CCF221J1CA-FC

R761	1W 4.7	RSE4R7J4ACCND	C129	50V 220P	CCF221J1CA-FC

R901	2W 15K	RSF153J5ACDND	C137	50V 100P	CCF101J1CA-FC

R911	2W 15K	RSF153J5ACDND	C138	50V 100P	CCF101J1CA-FC

R921	2W 15K	RSF153J5ACDND	C139	50V 15P	CCF150J1CA-FC

R931	1/2W 2.2	RSD2R2J4ABBND	C202	50V 0.01u	CKF103Z1FA-FC

C204	50V 0.01u	CKF103Z1FA-FC

C226	50V 0.01u	CKF103Z1FA-FC

Wire resistor	　	C301	50V 0.01u	CKF103Z1FA-FC

R513	5W 3.9	RWH3R9K3AK-NR	C303	50V 8P	CCF8R0D1CA-FC

R602  	RX27-3A	RWH1R8K5AF-NR	C304	50V 1000P	CKF102K1BA-FC

-5W-1.8		C318	50V 0.01u	CKF103Z1FA-FC

R603	5W 2.2K	RWH222J5AQ-NR	C403	50V 1000P	CKF102K1BA-FC

R609	2W 0.22	RWFR22K3AK-NR	C501	50V 0.01u	CKF103Z1FA-FC

C506	500V 3900P	CKP392K1BA-FC

C508	500V 1000P	CKP102K1BA-FC

Potentiometer	　	C522	2KV 470P	CKX471K2RL-ND

VR601	20KB	VDAAA203A001H	C600	2KV 470P	CKX471K2RL-ND

C603	1KV 1000P	CKW102K2BL-ND

C604	1KV 1000P	CKW102K2BL-ND

C605	1KV 1000P	CKW102K2BL-ND

Ceramic		C606	1KV 1000P	CKW102K2BL-ND

capacitor		C607B	2KV 1000P    	CKX102K2R L-ND

C103	50V 100P    	CCF101J1CA-FC	C609	2KV 470P	CKX471K2RL-ND

C104	50V 15P	CCF150J1CA-FC	C610	2KV 680P	CKX681K2RL-ND

C105	50V 15P    	CCF150J1CA-FC	C611	50V 100P	CCF101J1CA-FC

C107	50V 0.01u  	CKF103Z1FA-FC	C615  	~400V 2200P	CKMX222M302ND

C108	50V 0.01u  	CKF103Z1FA-FC	C616   	~400V 220P	CKMX221K302ND

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number	Ceramic		number	Electrolytic

capacitor                                              capacitor

C618	50V 4700P	CKF472K1BA-FC	C309	50V 10u	CEF100M1ACAFP

C621	1KV 470P	CKW471K2RL-ND	C317	25V 1000u    	CED102M1AMHNP

C623	1KV 470P	CKW471K2RL-ND	C320	50V 1u	CEF1R0M1ACAFP

C625	1KV 470P	CKW471K2RL-ND	C367	50V 10u	CEF100M1ACAFP

C627	2KV 470P	CKX471K2RL-ND	C368	50V 2.2u	CEF2R2M1ACAFP

C629	1KV 1000P	CKW102K2BL-ND	C404	35V 1000u	CEE102M1AMHNP

C641	50V 120P	CCF121J1CA-FC	C405	35V 100u	CEE101M1ACCFP

C698   	~400V 220P	CKMX221K302ND	C406	50V 1u	CEF1R0M1ACAFP

C706	50V 0.01u	CKF103Z1FA-FC	C408	50V 3.3u	CEF3R3M1ACAFP

C712	50V 0.01u	CKF103Z1FA-FC	C414	35V 1000u	CEE102M1AMHNP

C719	50V 100P	CCF101J1CA-FC	C502	25V 100u     	CED101M1AMBNP

C753	50V 16P	CCF160J1CA-FC	C504	50V 1u	CEF1R0M1ACAFP

C755	50V 1000P	CKF102K1BA-FC	C507	35V 47u	CEE470M1ACCFP

C777	50V 100P	CCF101J1CA-FC	C515	160V 2.2u	CEH2R2M1ACCFP

C903	50V 100P	CCF101J1CL-NC	C518  	160V10u	CEH100M1EMENP

C904	50V 470P	CKF471K1BL-NC	C607	400V 220u	CEM221M3AESNK

C913	50V 100P	CCF101J1CL-NC	C613	25V 100u    	CED101M1AMBNP

C914	50V 470P	CKF471K1BL-NC	C617	50V 33u	CEF330M1ACAFP

C923	50V 100P	CCF101J1CL-NC	C622	25V 2200u	CED222M2A MKNP

C924	50V 470P	CKF471K1BL-NC	C624	16V 470u	CEC471M1ACCFP

C941	2KV 2200P	CKX222K2BL-ND	C626	25V 1000u  	CED102M1AMHNP

C628  	250V220u  	CEH221M2EMLNP

Electrolytic		C630	35V 330u	CEE331M1AMFNP

capacitor		C632	16V 470u	CEC471M1ACCFP

C101	50V 1u	CEF1R0M1ACAFP	C635	50V 2.2u	CEF2R2M1ACAFP

C106	10V 470u	CEB471M1AMCNP	C636	10V 1000u	CEB102M1AMENP

C112	50V 2.2u	CEF2R2M1ACAFP	C645	16V 2200u	CEC222M1AMHNP

C120	16V 47u	CEC470M1ACAFP	C647	16V 47u	CEC470M1ACAFP

C123	25V 100u	CED101M1ACBFP	C701	50V 1u	CEF1R0M1ACAFP

C125	16V 2200u  	CEC222M1AMHNP	C702	50V 1u	CEF1R0M1ACAFP

C126	50V 10u	CEF100M1ACAFP	C703	50V 1u	CEF1R0M1ACAFP

C205	16V 100u	CEC101M1AMBNP	C704	50V 1u	CEF1R0M1ACAFP

C206	50V 1u	CEF1R0M1ACAFP	C705	50V 1u	CEF1R0M1ACAFP

C209	50V 1u	CEF1R0M1ACAFP	C707	16V 47u	CEC470M1ACAFP

C210	50V 0.47u	CEFR47M1ACAFP	C713	16V 220u	CEC221M1AMCNP

C213	50V 1u	CEF1R0M1ACAFP	C714	50V 2.2u	CEF2R2M1ACAFP

C218	50V 3.3u	CEF3R3M1ACAFP	C715	16V 100u	CEC101M1ACBFP

C219	16V 47u	CEC470M1ACAFP	C720	50V 1u	CEF1R0M1ACAFP

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number	Electrolytic		number

capacitor                                              Inductor

C752	50V 2.2u	CEF2R2M1ACAFP	L101   	 39uH-K	LBB390K----RG

C754	50V 0.47u	CEFR47M1ACAFP	L203	 10uH-K	LBB100K----RG

C757	50V 10u	CEF100M1ACAFP	L334    	 47uH-K	LAB470K1---NJ

C773	16V 470u	CEC471M1ACCFP	　	Non-standard

C774	16V 1000u	CEC102M1AMFNP	　	inductor

C811	50V 1u	CEF1R0M1ACAFP	FB501	M-COIL	LEA001-5---NJ

C812	50V 10u	CEF100M1ACAFP	FB502	M-COIL	LEA001-5---NJ

C931	250V 22u 	CEK220M1EMHNP	FB503	M-COIL	LEA001-5---NJ

C2001	50V 10u	CEF100M1ACAFP	FB610	M-COIL	LEA001-2---NJ

C2002	50V 10u	CEF100M1ACAFP	FB611	M-COIL	LEA001-2---NJ

FB612	M-COIL	LEA001-6---NJ

FB612A	6*5*1.5mm　	LEE004-----NJ

FB612B	6*5*1.5mm	LEE004-----NJ

Film capacitor		FB624A	6*5*1.5mm　	LEE004-----NJ

C111	100V 0.033u	CFG333K1-L-NW	FB624B	6*5*1.5mm　	LEE004-----NJ

C113	63V 0.1u	CAY104K1-K-FJ	FB625A	6*5*1.5mm　	LEE004-----NJ

C201	100V 0.022u	CFG223K1-L-NW	FB625B	6*5*1.5mm　	LEE004-----NJ

C207	63V 0.1u	CAY104K1-K-FJ	L502	M-COIL	LEA001-3---NJ

C302	100V 0.01u	CFG103J1-L-NW	LW545	M-COIL	LEA001-3---NJ

C305	100V 0.01u	CFG103J1-L-NW

C319	100V 4700P	CFG472K1-L-NW

C401	63V 0.47u	CAY474K1-L-NJ	　	Linearity filter

C402	50V 0.22u	CAY224K1-L-NJ	L601	Linearity filter	LG-003-----NL

C407	100V 0.1u	CFG104K1-L-NW

C409	100V 0.022u	CFG223K1-L-NW

C505	100V 0.015u	CFG153J1-L-NW

C509	100V 0.056u	CFG563K1-L-NW	　	Horizontal linearity

C511	1600V 9100P	CMZ912J8-K-NS	L501  Horizontal linearity correcting coil  LF-015-2---NJ

C514	250V 0.39u	CMK394J8-K-NS

C519	63V 0.1u	CAY104K1-K-FJ

C601	~275V 0.22u	CMNX224M801NP

~275V 0.22u	CMNX224M801NS	　	Inter medium frequency coil

C608	~250V 0.047u	CMKX473K401NE	L202    Matching inter	LTC001-N---NV

C612	63V 0.1u	CAY104K1-K-FJ	medium frequency

C636B	63V 0.1u	CAY104K1-K-FJ	L204	45.75MHZ	LTC020-C---NV

C642	63V 0.1u	CAY104K1-K-FJ

C643	100V 0.033u	CFG333K1-L-NW	　	Themistor (PTC)

C750	100V 0.047u	CFG473K1-L-NW	PS601	MI73-18	DH2180M001-NC

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number			number

LED                                                 Diode

VD112	FG313003	DLB52004---NA	VD622	RGP10D	DRRGP10D-01ND

VD623	RGP10D	DRRGP10D-01ND

Diode		VD624	BYW95C	DRBYW95C---BP

VD102    MTZJ5R6B	DZMTZJ5R6B-RR	VD625	RGP10D	DRRGP10D---ND

VD103	MTZJ3.6A	DZMTZJ3R6A-RR	VD633	MTZJ15C	DZMTZJ15C--RR

HZ4A2	DZHZ4A2---RH	VD641	RGP10J	DRRGP10J-01ND

VD104	1N4148-T77	DR1N4148---RR	VD643	1N4148-T77	DR1N4148---RR

1N4148-T77	DR1N4148---RH	　	1N4148-T77	DR1N4148---RH

VD105	1N4148-T77	DR1N4148---RR	VD644	RGP10D	DRRGP10D-01ND

1N4148-T77	DR1N4148---RH	VD645	MTZJ6R2C	DZMTZJ6R2C-RR

VD106	MTZJ3.6A	DZMTZJ3R6A-RR	　	HZ6C3	DZHZ6C3----RH

HZ4A2	DZHZ4A2---RH	VD702	1N4148-T77	DR1N4148---RR

VD107	MTZJ5R6B	DZMTZJ5R6B-RR	　	1N4148-T77	DR1N4148---RH

VD108	1N4148-T77	DR1N4148---RR	VD703	1N4148-T77	DR1N4148---RR

1N4148-T77	DR1N4148---RH	　	1N4148-T77	DR1N4148---RH

VD111	1N4148-T77	DR1N4148---RR	VD704	1N4148-T77	DR1N4148---RR

1N4148-T77	DR1N4148---RH	　	1N4148-T77	DR1N4148---RH

VD114	MTZJ5.1B	DZMTZJ5R1B-RR	VD706	1N4148-T77	DR1N4148---RR

HZ5C1	DZHZ5C1---RH	　	1N4148-T77	DR1N4148---RH

VD2001	RGP10D	DRRGP10D-01ND	VD721	MTZJ7.5C	DZMTZJ7R5C-RR

VD2002	MTZJ12A	DZMTZJ12A--RR	　	HZ7C2	DZHZ7C2----RH

HZ11C2	DZHZ11C2---RH	VD809	MTZJ5.1B	DZMTZJ5R1B--RR

VD401	RGP10D	DRRGP10D-01ND	　	HZ5C1	DZHZ5C1---RH

VD501	MTZJ8.2C	DZMTZJ8R2C-RR	VD981	1N4148	DR1N4148---NR

HZ9A3	DZHZ9A3----RH	　	1N4148	DR1N4148---NH

VD504	RGP10J	DRRGP10J-02ND

VD525	BYD33G-T	DRBYD33G---BP	　	SAW Filter

VD603	TTVR4J	DRTTVR4J-01NZ	FL201	M1967	XFS031-----NB

VD604	TTVR4J	DRTTVR4J-01NZ

VD605	TTVR4J	DRTTVR4J-01NZ	　	Crystal oscillator

VD606	TTVR4J	DRTTVR4J-01NZ	X701	3.58MHz	XX-012---01NL

VD609	1N4148-T77	DR1N4148---RR	X101	32KHz	XX-015-----NW

1N4148-T77	DR1N4148---RH

VD610	RGP10D	DRRGP10D-01ND	　	Power switch

VD611	RGP10J	DRRGP10J---NG	SW601	KDC-A04-2	KPW12HL011--D

VD612	RGP10J	DRRGP10J---NG

VD614	RGP10D	DRRGP10D-02ND	　	Fuse

VD621	RGP15D	DRRGP15D-01ND	F601	250V  3.15A	FBA3154S02C-A

Position	   Type	    Parts no.	Position	   Type	    Parts no.

number			number

Transistor                                             Push switch

V102	2PA1015G	Q2PA1015-GNF-	SW101	7.5mm	KTA11VA004--G(Y)

V103	2PC1815G	Q2PC1815-GNF-	SW102	7.5mm	KTA11VA004--G(Y)

V107	2PC1815G	Q2PC1815-GNF-	SW103	7.5mm	KTA11VA004--G(Y)

V108	2PA1015G	Q2PA1015-GNF-	SW104	7.5mm	KTA11VA004--G(Y)

V114	2PC1815G	Q2PC1815-GNF-	SW105	7.5mm	KTA11VA004--G(Y)

V203	2PA1015G	Q2PA1015-GNF-	SW106	7.5mm	KTA11VA004--G(Y)

V302	2PC1815G	Q2PC1815-GNF-

V501	KSC23830	QKSC2383-0NF-	　	Socket connector

V502	KSD5702	QKSD5702--BN-	XS601	TJC1-2A	GAAV2001----Y

V604	2PC1815G	Q2PC1815-GNF-	HE	TJC2-1A	GABV1002----Y

V613	STP6NC60	QSTP6NC60--N-	XS602	TJC2-2A	GABV2001----N

V642	MCR22-6	QMCR22-6---G-	DY	TJC2-5A	GABV5001----Y

V643	2PC1815G	Q2PC1815-GNF-	XS151	TJC3-2A	GACV2001----Y

V701	2PC1815G	Q2PC1815-GNF-	KP	TJC3-5A	GACV5001----Y

V901	2SC2482	Q2SC2482--NN-	XS3101	TJC3-6A	GACV6001----Y

KSC2330		KQ	TJC3-6A	GACV6002----Y

V911	2SC2482	Q2SC2482--NN-

KSC2330

V921	2SC2482	Q2SC2482--NN-	XS901	CRT Socket	GBAVL007----A

KSC2330

XS801	AV terminal	GBBH2-02----Y

IC

VD110	CW574CS	NCW574CS---N-	TPAGC	Test probe	GABV1001----Y

N103	HS0038B	NHS0038B---N-

V640	KA431Z	NKA431Z----F-

N602	KA7805	NKA7805----N-		Transformer

N603	KA7812	NKA7812----N-	T501    Horizontal drive	TD-0004----0L

N102	KS24C04I	NKS24C04---N-I	T502    Fly Back Transformer    TF-0058----1D

24LC04BP	N24LC04BP--N-	T601    Power supply transformer  TM-0047----0L

AT24C04	NAT24C04---N-

N201	LA76814	NLA76814---N-		　FS Tuner

N401	LA7840	NLA7840----N-	A101  Frequency combined screw thread  BXATB007F---H

N101	LC863232A-5V16	NLC863232A-NA

N604	LTV817C	NLTV817C---N-

N601	MC44608P75	NMC44608P75N-	F501A	Fuse clamp	MB0X0011---G8

N304	TDA7056B	NTDA7056B--N-	F501B	Fuse clamp	MB0X0011---G8

3. Cabinet Part list and Installation of Housing

The part list and structural installation of cabinet of Type 2188 and 2168 are respectively described as follows:

3.1 The part list and structural installation of cabinet of Type 2188

Partial list of Type 2188 Cuban cabinet

- Front Case

- Loudspeakers (2)

Tap screws ST4X12 (8)

- Trademark

- Elastic engaged ring (2)

- Combined button

Tap screws ST3X12 (2)

- Lens

- Transparent window

- Optical conductor

Tap screws ST3X12 (2)

- Power switch

- Press spring

- Supported blocks M6 (4)

Tap screws ST4X20 (12)

- Nuts with quincunx washer（4）

- combined nuts（4）

- Support of Fly Back Transformer

Tap screws ST4X12 (1)

Tap screws ST4X16 (1)

- Rear cover

- Mark Label on rear cover

Structural installation

|  | Rear cover Remove 1. Pull out power cord from power socket. 2. Remove nine screws as the figure 1 shown. 3. Gently pull out the rear cover from the front cabinet. Chassis Remove 1. Remove the rear cover. 2. Discharge the EHT mouth of tube at first, and remove the EHT cap, gently pull out the CRT board, pull out the degauss coil socket, deflecting wire plug, loudspeaker plug, grounding conductor socket of tube, and clips of power cord. 3. Carefully pull out the chassis from the front cabinet. Tube Remove Warning: Don’t move the deflecting coil and magnetic body on the neck of tube, must carefully keep those parts in good condition, except the CRT is damaged. The CRT surface discharge should be made before disassembling. 1. Remove chassis. 2. Put the front cabinet with tube downward, lay it down on a soft table-board. 3. Unscrew four combined nuts on ear-ring of tube with a special socket spanner, carefully handle out the tube from front cabinet. 4. Install the tube according to the opposite procedure of that mentioned above. properly install the degauss coil and grounding conductor of tube onto the tube as fig. 2 shown. |
| --- | --- |

3.2 The part list and structural installation of cabinet of Type 2168

Partial list of Type 2168 Cuban cabinet

- Front cabinet

- Loudspeakers (2)

Tap screws ST4X12 (8)

- Trademark

- Elastic engaged ring (2)

- Combined button

Tap screws ST3X10 (3)

- Guide light beam

Tap screws ST3X10 (2)

- Transparent window

- Power switch

- Press spring

- Rubber gasket (4)

- Metal gasket (4)

- Combined nuts5X40（4）

- Rear cover

- Support of high voltage package

Tap screws ST4X12 (1)

Tap screws ST4X16 (1)

- Hook of power cord

- Mark label on rear cover

Structural installation

|  | Rear cover Remove 1. Pull out power cord from power socket 2. Remove 10 screws as the fig. 1 shown. 3. Gently pull out the rear cover from the front case. Chassis disassembly 1. Remove the rear cover. 2. Discharge the EHT mouth of tube at first, and remove the EHT cap, gently pull out the CRT board, pull out the degauss coil plug, deflecting wire socket, loudspeaker plug, grounding conductor socket of tube, and clips of power cord. 3. Carefully pull out the Chassis from the front case. CRT disassembly Warning: Don’t move the deflecting coil and magnetic body on the neck of tube, must carefully keep those parts in good condition, except the CRT is damaged. The CRT surface discharge should be made before disassembling. 1. Remove chassis. 2. Put the front case with tube downward, lay it down on a soft table-board. 3. Unscrew four combined nuts of ear-ring on tube with a special socket spanner, carefully handle out the tube from front case. 4. Install the tube according to the inverse procedure of that mentioned above. properly install the degauss coil and grounding conductor of tube onto the tube as fig. 2 shown. |
| --- | --- |

4. Signal flow Diagram

This TV set Consist of main chip LA76814 controlled by Sanyo I2C bus and CPU LC863232A .

LA76814 is IC dedicated for NTSC, integrated following circuits inside: amplifying and detecting in intermedium frequency, AFC, demodulating of sound intermedium frequency, brightness delay-line, filtering and demodulating of color signal, processing of horizontal-vertical small signals.

TDA7056B is used for power amplifier of sound signal.

LA7840 is used for vertical power amplifier.

The detailed signal flow diagram is shown as follows:

V IN          EXT AUDIO IN

RF AGC

VEDIO OUT        EXT V IN

SELECT V OUT

N201 LA76814

SECAM   R-Y  IN

SECAM  B-Y  IN

SDA

SCL

RGB_IN           R/G/B/OUT

AUDIO OUT

V OUT   FBP IN    H OUT

BAND1

BAND2

N101 CPU

LA8632XXX

SDA

SCL

R/G/B OUT

H   V

TUNER

SAW

CRT  PCB

CRT

A I

V I

H IN

H OUT

V IN

N401

LA7840

DY

SCREEN/

FOCUS

EHT

117V

POWER

UNIT

180V

15V

26V

5V

N304

TDA7056B

Signal flow diagram

### Signal block diagram

5. Adjustment Test

5.1 I2C Bus Adjustment Test

5.1.1 Method of access bus adjustment test menu

Using remote controller, press “MUTE”, at same time presses “MENU” on Front cabinet “TEST” characters can display on the TV set screen, i.e. the bus adjustment status has been entered, then press “0”~”9” numeric key on the remote controller, and into corresponding adjustment menu. After adjustment, exit the adjustment status according the same method. In addition, in case of normal use of TV set, short-circuited the jumper W325 and W326 can enter the status “TEST”.

5.1.2 Explanation of adjustment test menu

[M0] MENU

MENU0

TEST SIGNAL  		0

SOUND TRAP	  	0

CPU VOLUME 		1

WOOFER   			OFF

◎ When enter MENU0 adjustment test menu, CPU will automatically

set “Picture” to status of “Standard”, and close “Picture enhancement” setting.

Description:

TEST SIGNAL=0, receive TV signals.

TEST SIGNAL=1, receive white signals produced inside of CPU.

TEST SIGNAL=2, receive gray signals produced inside of CPU.

TEST SIGNAL=3, receive black signals produced inside of CPU.

TEST SIGNAL=4, receive pane and cross signals produced inside of CPU.

SOUND TRAP used for fine adjustment of sound trap frequency.

WOOFER for woofer control, ON is available.

[M1] MENU

MENU1

LA76814			ON

PAL_M			OFF

PAL-N

OFF

NTSC			OFF

ENGLISH			ON

ESPANNOL		ON

PORTUGUES		ON

◎ MENU 1 setting

·When enter MENU1 adjustment test menu, CPU will automatically

set “Picture” to status of “Standard”, and close “Picture enhancement” setting.

Description:

·LA76814: when LA76814 used as main chip, set this item to ON.

·PAL-M/PAL-N/NTSC: When LA 76812 used as main chip, the 
color system is selected.

·ENGLISH/ESPANOL/PORTUGUES: for screen language

selection: English/Espanol/ Portuguese.

[M2] MENU

◎ MENU 2 setting

·When enter MENU2 adjustment menu, CPU will automatically
set “Picture” to status of “Standard”, and close “Picture enhancement” setting .

MENU2

AV2				OFF

S-VIDEO			OFF

START-ON			OFF

OSDCONT			1

WIDE				20

ZOOM			95

Description:

·AV2: set ON to available.

·S-VIDEO: set ON to available.

·OSD-CONT: Set the contrast of OSD.

·WIDE/ZOOM: set the size of zoom.

[M3] MENU

◎ MENU3 for vertical and horizontal circuits adjustment test, about
the method of adjustment test, see “5 Complete set adjustment test”.

MENU3

H-PHASE			11

V.POSI			28

V.SIZE        		73

V.LIN          		13

V.SC       			5

OSD.H.DISPLAY		20

·H-PHASE: adjustment test item of picture horizontal center.

·V-POSI: adjustment test item of picture vertical center.

·V-SIZE: adjustment test item of picture vertical amplitude.

·V-LIN: adjustment test item of picture vertical scan Non-linearity.

·V-SC: adjustment test item of picture vertical S calibration.

·OSD H DISPLAY: adjustment test item of D horizontal display
position .

MENU4

RF-AGC             	45

[M4] MENU

◎MENU4 for RF-AGF adjustment test, about the method of 
adjustment test, refer to “5 Complete set adjustment test”.

·RF AGC: for RF-AGC adjustment test.

MENU5

CL				32

BT				48

CT				48

SC

RB				50

GB				50

BB				50

RD				60

GD				9

BD				60

SB				90

[M5]MENU

◎MENU5 for adjustment test of Screen/White balance/Sub 
brightness, about the method of adjustment test, see “5 Complete set 
adjustment test”.

·When enter MENU5 adjustment test menu, CPU will automatically
set “Picture enhancement” to status of “closed”, color saturation to 0, 
brightness to 50.

·CLColor saturation control

BTBrightness control

CTContrast control

SBSub-brightness

·RB/GB/BBCutoff of red/green/blue gun

·RD/GD/BDExcite of red/green/blue gun

MENU6

R/B BALANCE         7

R/B ANGLE           	7

[M6]MENU

◎MENU6 for setting of color decode item.

·When enter MENU6 adjustment test menu, CPU will automatically 
set “Picture” to status of “Standard”, and close “Picture enhancement” 
setting.

·R/B BALANCER/B balance

R/B ANGLER/B decode and demodulation angles.

MENU7

H blank L           4

H blank R           4

V.COMP            7

H.Freq            24

FBP.BL.SW        1

[M7]MENU

◎MENU7

·When enter MENU7 adjustment test menu, CPU will automatically 
set “Picture” to status of “Standard”, and close “Picture enhancement”.

·H blank LHorizontal blanking (left)

H blank RHorizontal blanking (right)

V.COMP(Vertical size compensation) Compensating vertical size influenced by contrast.

H.FreqHorizontal frequency (It is unnecessary to adjust in batch production.)

FBP.BL.SWSelect working manner of horizontal blanking.

0=Selected IC inside horizontal blanking

1=Selected IC both inside and outside FBP horizontal blanking.

[M8]MENU

MENU8

SUBBASS			42

SUBTREBE			42

LOGOTIME			12

◎MENU8 for parameters setting

·When enter MENU8 adjustment test menu, CPU will automatically 
set “Picture” to status of “Standard”, and close “Picture enhancement”.

·SUB BASSSub-bass

SUB TREBLESub-treble

LOGO TIMESetting of LOGO display duration time

[M9]MENU

◎MENU for parameters setting

·When enter MENU9 adjustment test menu, CPU will automatically 
set “Picture” to status of “Standard”, and close “Picture enhancement”.

MENU9

FMGAIN			0

VIDEOLEVEL		7

OSDCONT			127

SUBCOLOR			127

SUBCOLOR			127

SUBSHARP			63

SUPTINT			51

BRIABLTH			4

BRIABL			0

·FM GAIN Sound frequency discrimination gain

VIDEO LEVEL Video detection output level

OSD CONSTANT OSD contrast

SUB COLOR NTSC system sub-color

SUB COLOR PAL system sub-color

SUB SHARP Sub-sharpness

SUB TINT Sub-tint

RBI ABL TH Adjustment of ABL initial control position

BRI ABL 1=ABL unable

0=ABL enable

5.1.3 Preset of I2C bus adjustment test item (Preset related to CRT used)

| MI | Item | Range | Preset | Note | MI | Item | Range | Preset | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M0 | TEST.SIGNAL | 0～4 | 0 |  | M5 | CL | 0～63 | 32 |  |
|  | SOUND .TRAP | 0～7 | 0 |  |  | BT | 0～63 | 48 |  |
|  | CPU VOLUME | 0～1 | 1 |  |  | CT | 0～127 | 48 |  |
|  | HEADPHONE | On/Off | Off |  |  | SC |  |  |  |
|  |  |  |  |  |  | RB | 0～255 | 30 |  |
| M1 | LA76814 | On/Off | On |  |  | GB | 0～255 | 80 |  |
|  | PAL-M | On/Off | Off |  |  | BB | 0～255 | 40 |  |
|  | PAL-N | On/Off | Off |  |  | RD | 0～127 | 80 |  |
|  | NTSC | On/Off | On |  |  | GD | 0～15 | 15 |  |
|  | English | On/Off | On |  |  | BD | 0～127 | 80 |  |
|  | Espannol | On/Off | On |  |  | SB | 0～127 | 90 |  |
|  | Portugues | On/Off | On |  |  |  |  |  |  |
|  |  |  |  |  | M7 | H BLANK L | 0～7 | 4 |  |
| M2 | AV2 | On/Off | Off |  |  | H BLANK R | 0～7 | 4 |  |
|  | S-Video | On/Off | Off |  |  | V.COMP | 0～31 | 7 |  |
|  | START.ON | On/Off | Off |  |  | H.FREQ | 0～63 | 24 |  |
|  | OSD.. CONT | 0～3 | 1 |  |  | FBP.BLK.SW | 0～1 | 0 |  |
|  | Wide | 0～127 | 20 |  |  |  |  |  |  |
|  | Zoom | 0～127 | 95 |  | M8 | SUB BASS | 0～63 | 42 |  |
|  |  |  |  |  |  | SUB TREBLE | 0～63 | 42 |  |
| M3 | H-PHASE | 0～31 | 11 |  |  | LOGO TIME | 0～60 | 12 |  |
|  | V. POSI | 0～63 | 28 |  |  |  |  |  |  |
|  | V. SIZE | 0～127 | 73 |  | M9 | FM GAIN | 0～1 | 0 |  |
|  | V. LIN | 0～31 | 13 |  |  | VIDEO LEVEL | 0～7 | 7 |  |
|  | V.SC | 0～31 | 5 |  |  | SUB CONT | 0～127 | 127 |  |
|  | OSD.H.display | 0～63 | 20 |  |  | SUB COLOR | 0～127 | 127 | NTSC |
|  |  |  |  |  |  | SUB COLOR | 0～127 | 127 | PAL |
| M4 | RF AGC | 0～63 | 20 |  |  | SUB SHARP | 0～63 | 63 |  |
|  |  |  |  |  |  | SUB TINT | 0～127 | 51 |  |
| M6 | R/B Balance | 0～15 | 7 |  |  | BRI ABL TH | 0～7 | 4 |  |
|  | R/B Angle | 0～15 | 7 |  |  | BRT ABL | 0～1 | 0 |  |

5.2 Complete set adjustment test method

5.2.1 Set main power supply (+B)

Receive color bar signals, and set “Picture” to “Standard” status. Adjusting VR601, let +B voltage = 1170.5 V.

5.2.2 IF adjustment test

- Connect 45.75 MHz 100 dB signal to TUNER IF test point.

- Measure voltage of N201/PIN47.

- Adjusting L204, let V47=3.40.1 V.

5.2.3 I2C bus adjustment test method

1. MENU3 for horizontal and vertical scan circuit adjustment, receive the signals of black-white test card (numeric card).

- Adjusting H-SHIFT value, let the picture horizontal center align at horizontal center of tube screen.

- Adjusting V-POSI value, let the picture vertical center align at vertical center of tube screen.

- Adjusting V-SIZE value, let the picture vertical amplitude conform to requirement.

- Adjusting V-LIN value, let the picture vertical linearity conform to requirement.

- Adjusting V-SIZE value, let the heights of top and bottom panes of picture equal to that of the center pane.

- Adjusting OSD H DISPLAY value, let position of display bars (with number figures at side) is symmetrical about right and left sides.

2. MENU4 for RF AGC adjustment

Receiving 60 dBu (1 mV) color bar signals, adjust AGC value (the voltage from high to low), let the picture noise gradually reduced to level just aware.

3. MENU5 for adjustment of CRT cutoff, dark and Bright balance, and sub-brightness

After pre-heating the TV set fully, receive gray-white two-step signals.

- Adjusting BT (brightness) value and CT (contrast) value, of FBT let the brightness of white level and gray level of picture conform to conventional requirement of adjustment of manufactory.

- Select option to SC and press V+, then adjust Screen potentiometer of FBT, let a dim mono-color (one color of red, green and blue, sometimes may lean to yellow, violet or white) horizontal light line display on screen. Remember this color, press the numeric key 1, 2, 5, 6, 9, 0, let the horizontal light line become to white, then adjust SCREEN potentiometer again, let the horizontal light line become to just can visible.

- Select option to RB, GB and BB, and adjust RB, GB and BB values, let the gray color temperature meet the requirement.

- Select option to RD, GD and BD, and adjust RD, GD and BD values, let the white color temperature meet the requirement.

- Repeat c) and d) steps, let both gray and white temperature simultaneously meet the requirement.

- Receiving black-white 8-step signals, adjust SB (sub-brightness) value, let the difference between first step and second (the most dim) step can just distinguish.

3.2.4 Checking x-ray protective circuit

- Short-circuit jumperTP2001 and TP2002, activate protective circuit.

- Remove the short circuit wires, and then press CH+ or CH- key, TV set startup again.

6. Trouble shooting

14

13

12

11

8

10

8

Audio Power

Amplifying

14V

Rectifying and

Detection

Rectifying and

Detection

Rectifying and

Detection

15V

Rectifying and

Detection

N201

T611

10V

Rectifying and

Detection

N602 / 5V

N603 / 12V

CPU
Power Supply

26V

H.Dri
Power Supply

V.OUT 
Power supply

117V

H.OUT
Power Supply

Complete set power supply outline

6.1 No raster, no picture, no sound

General, these failures are produced by power sources, because of which refer to a wider area, so that can be divided them two conditions to explain: no +B 117 V and existing +B 117 V.

(1) No +B(117 V) voltage (voltage of network 110 V ac)

These failures may be caused by switching type power. Because of the TV set has no-load protection, over-load protection, over-current protection, once these protections are activated, the CPU will enter to “STAND BY” status. So that status confuses with conventional CPU standby, some difficulties are occurred for failure estimation. When the switching type power supply operating normally +B=117 V, but +B=17 V when come into standby status.

Check voltage+B

Refer to the no operation failures of CPU

=0V

=17V

Check fuse F601 or R602

Y

N

Check voltage on D pole of V613
 (switching transistor)

Check C601,

C603~C606, L601,

V613, V502 at

condition of

short-circuit?

=140V

Check 110 V ac input and VD603~VD606 rectifying, 
V613.

Y

Replace the 
parts of 
failure

(2) Existing +B 117 V

These failures may be caused by abnormal operation of horizontal scan circuit, to confirm which can observe the filament of tube whether bright on. If determined horizontal scan in abnormal operation, look for the failure start from the horizontal drive stage. Search upward to LA76814 with method of checking dc voltage and waveform according to sequence: N201 PIN27/H.OUT port N201 PIN25 power supply N603/15 V output N101CPU/PIN42(STAND BY) output.

Check voltage at pin of V501(C) =19 V

N

Y

=26V

18V

Check V505B pole and N201/PIN27 waveform

Check V501 horizontal drive stage

Y

Check filament circuit of FBT~CRT

Check horizontal output stage and deflection circuit

N

Check corresponding parts

Check N201/PIN25=5V
(input current at this pin is 10 mA)

Check N201

Y

N

Check voltage at N101/PIN16=0 V

Check voltage at V602E, V602C

N

=2.3V

Y

After restart TV, check voltage of negative pole of VD2002 <10 V

Check CPU according to non- operation

N

Check N101/CPU/PIN42=5 V

Check CPU/PIN42 STANDBY status

N

10V

Y

Check voltage of main power supply whether too high or value of C511C512 abnormal

Check VD2002

Y

Check V604

6.2 Existing raster and no picture

Because of the picture video signals and OSD characters display are input to PIN44, PIN15, PIN16, PIN17 of LA76814(N201), so to observe characters on screen of faulted TV can determine the approximate area of fault.

Display OSD characters on screen

N

Y

When searching automatically channels, check CVBS waveform existed at N201/PIN(46)

Check 180 V power supply on CRT board

Y

N

N

Y

Check 180 V power supply of relative plug-in connector

Check CVBS waveform existed at N201/PIN44

Check operating status of TUNER

Check bus between N101, N102, N201*

Y

N

Check FBP waveform at N201/PIN28

Check external element connect to N201/PIN28

Check circuit between PIN(46)~(44) of N201

Check  CVBS waveform at N201/PIN19/PIN20/ PIN21

N

Y

*：Whenever open or interconnect on the bus from CPU to E2PROM and main chip, no picture can occur.

Check N201

Check CRT board

6.3 Existing raster, no picture and sound

Processing according “6.2 Existing raster and no picture” at first, let the picture display normally, then check sound. About the repair method, see “6.4 Existing raster and picture, no sound”.

6.4 Existing raster and picture, no sound

Check waveform at two pins of loudspeaker whether normal

Y

N

Check waveform at N304/PIN(3 ) whether normal

Check waveform at N304/PIN(6)/PIN(8) whether normal

Y

N

Y

N

Check waveform at N201/PIN1 whether normal

Check voltage of volume control at N304/PIN(5)

Check external element connected to N304/PIN(6)/ PIN(8)

Check the loudspeaker and its connection

Y

Y

N

Check mute circuit of CPU/VOL and V108

Check N304

Check elements between N201(1)/PIN1 to N304(3)/PIN

Check SIF circuit at N201/PIN52/PIN54(2) / SIF

N

6.5 CPU non-operating

Check voltage at N101/PIN(42)
 (STANDBY) =5V

Y

N

Check voltage at N101/PIN(16) (X-RAY) =0 V

Check V604

Y

N

When restart TV, check voltage on negative pole VD2002=9.4 V

Check voltage at N101/PIN(12) =5V

Y

N

Check voltage at N101/PIN(17 )(RESET)=5V

Check 15V~CPU 5V Supply

10V

Y

Check
C511
C512

Check protection circuit of V605 and VD2001

Y

N

Check V102 RESET circuit

Manual RESET

Y

N

Check oscillating waveform at N101/PIN(10)/PIN(11)

Check crystal oscillator and external elements

N

Y

Check functional inching  key of the TV whether pressed

Check N101

N

6.6 Cannot save channels

When searching automatically channels, observe whether can display picture

Y

N

Check BT voltage of TUNER=30 V

Check SD voltage variation at CPU/PIN(33)

Y

Y

N

N

Check AFT voltage variation at CPU/PIN(15)

Check voltage variation at N201/PIN(22)

Check TUNER

Check R104 and VD101

N

Check 33 V of voltage

7. Overview of IC function

7.1 N801 LC8632XXX  Function: microprocessor

| Symbol | PIN | Function description | Symbol | PIN | Function description |
| --- | --- | --- | --- | --- | --- |
| NC | 1 | No connected | STANDBY | 42 | STAND BY control output |
| NC | 2 | No connected | TV/AV | 41 | TV/AV control output |
| NC | 3 | No connected | AV1/AV2 | 40 | AV1/AV2/S-VHS control output |
| STOP | 4 | Bus control port | MUTE | 39 | MUTE output |
| NC | 5 | No connected | SPOT KILLER | 38 | Blanking as TV turn off |
| VOLUME | 6 | PWM output of volume control | NTSC3.58 | 37 | No connected |
| NC | 7 | No connected | BAND2 | 36 | No connected |
| NC | 8 | No connected | BAND1 | 35 | No connected |
| GND | 9 | Ground | IR-IN | 34 | Remote signal input |
| XTAL1 | 10 | 32kHz crystal oscillator | SD | 33 | Identification signal input |
| XTAL2 | 11 | 32kHz crystal oscillator | SYS IN2 | 32 | No connected |
| VDD | 12 | Connected to 5 V power supply | SYS IN1 | 31 | No connected |
| KEY1 | 13 | Keyed input 1 | NC | 30 | No connected |
| KEY2 | 14 | Keyed input 2 | NC | 29 | No connected |
| AFT | 15 | AFT voltage input | SCL | 28 | EEPROM SCL |
| X-RAY | 16 | X-RAY protection input | SDA | 27 | EEPROM SDA |
| RESET | 17 | High level: Reset | NC | 26 | No connected |
| FILT | 18 | Filtering | OSD BLANK | 25 | OSD blanking output |
| CATV | 19 | CVBS signals input | B OUT | 24 | OSD B output |
| V.SYN | 20 | Vertical pulse input | G OUT | 23 | OSD G output |
| H.SYN | 21 | Horizontal pulse input | R OUT | 22 | OSD R output |

7.1.1 Description of pins of LC8632XXX

·Pin No.1: NC

·Pin No.2: S-VHS selector switch. In case of the S-VHS status, the level is low, in other cases the level is high.

·Pin No.3: NC

·Pin No.4: CPU bus control port, in normal operating the level is high, when this level is forced in low level, CPU bus will be opened, general, it is used for automatic adjustment of white balance.

·Pin No.5: NC

·Pin No.6: PWM output of volume control

·Pin No.7: NC

·Pin No.8: NC

·Pin No.9: GND

·Pin No.10 and 11: 32 kHz oscillator

·Pin No.12: VDD (5V)

·Pin No.13:  Key input

| Function | MENU | TV/AV | POWER | V+ | V- | P+ | P- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage | 0 | 0.625 | 1.25 | 1.875 | 2.5 | 3.125 | 3.75 |

·Pin No.14: TEST function input

·Pin No.15: AFT voltage input

·Pin No.16: X-RAY protection input (valid for high level)

·Pin No.17: Reset (valid for high level)

·Pin No.18: filtering

·Pin No.19:Full TV signals input, used for CCD Sampling signal.

·Pin No.20: Vertical pulse input

·Pin No.21:Horizontal pulse input.

·Pin No.22, 23, 24: R, G, B screen display output

·Pin No.25: OSD blanking output

·Pin No.26: NC

·Pin No.27: I2C data bus (SDA0) port

·Pin No.28: I2C clock bus (SCL0) port

·Pin No.29: SDA1 (no used)

·Pin No.30: SCL1 (no used)

·Pin No.31: NC

·Pin No.32: NC

·Pin No.33: Identification signal input

Note: If no signal at TV mode, SD (PIN33) is at low level, after 10 seconds, CPU will enter to status of standby. In other conditions (Video, S-Video), this status cannot occur.

·Pin No.34: Remote signal input

·Pin No.35 and 36: NC

·Pin No.37: NC

| ·Pin No.38: Blanking output as TV turn off. Only when turn-off power switch, it outputs high level. MODE | CPU | PIN NO. |  |
| --- | --- | --- | --- |
|  | PIN40 | PIN41 | PIN2 |
| TV | H | H | H |
| AV | H | L | H |
| （AV1） |  |  |  |
| AV2 | L | H | H |
| S-VHS | L | L | L |

TV/AV Truth-value table

·Pin No.39: Sound MUTE output. In normal at low level, when switching channel or in case of no signal, it outputs high level.

·Pin No.40 and 41: Control signal output of TV/AV(AV1)/AV2/S-VHS etc.

·Pin No.42: STAND BY output. In normal condition at high level, in STAND BY status at low level.

7.1.2 Keys of the TV set: MENU, TV/AV, VOL-, VOL+, CH-, CH+

1) Continuously press [MENU] key, the menu will change as follows:

PICTURE      SETUP        PIC-ENH     PRESET     FAV

NORMAL

a). TV mode

PICTURE  SETUP PIC-ENH  Normal operation

b) AV mode

7.2 N201 LA76814  Function: I2C bus control and NTSC small signal processing

| Symbol | PIN | Function description | Symbol | PIN | Function description |
| --- | --- | --- | --- | --- | --- |
| AUDIO OUT | 1 | Sound frequency output | FBP IN | 28 | Sandcastle pulse forming |
| FM OUT | 2 | Sound detection output | VCD I REF | 29 | Reference current for horizontal frequency |
| PIF AGC | 3 | Inter medium frequency amplifying AGC | CLOCK OUT | 30 | 4MHz clock output |
| RF AGC OUT | 4 | Radiation frequency amplifying AGC | NC | 31 | No used |
| PIF IN | 5 | Picture intermedium frequency input | OSD FIL | 32 | OSD circuit filtering |
| PIF IN | 6 | Picture intermedium frequency input | GND（CCD/H） | 33 | GND (IH_DL) |
| GND（ IF ） | 7 | GND (intermedium frequency amplifying) | GND（X-RAY） | 34 |  |
| VCC（VIF） | 8 | Power supply (intermedium frequency amplifying) | NC | 35 | No used |
| FM FIL | 9 | Sound detection filtering | APC FIL | 36 | Color APC filtering |
| AFT OUT | 10 | AFT output | ACC FIL | 37 | ACC filtering |
| BUS DATA | 11 | Data bus | XIAL | 38 | 3.58MHz Crystal oscillator |
| BUS CLOCK | 12 | Clock bus | NC | 39 | No used |
| ABL | 13 | ABL | SELECT V OUT | 40 | Output of selection of internal/external video |
| R IN | 14 | Character red input | GND（V/C/D） | 41 | GND (video/color/deflection) |
| G IN | 15 | Character green input | EXT V IN | 42 | External video signal input |
| B IN | 16 | Character blue input | VCC（V/C/D） | 43 | Power supply (video/color/deflection) |
| BL IN | 17 | Character blanking input | INT V IN | 44 | Internal video signal input |
| VCC（RGB） | 18 | Power supply (RGB) | BLACK STR | 45 | Black level extend filtering |
| R OUT | 19 | Red output | VIDEO OUT | 46 | Video detection output |
| G OUT | 20 | Green output | APC FIL | 47 | Video detection APC filtering |
| B OUT | 21 | Blue output | VCO COIL | 48 | Video detection |
| SYNC | 22 | Synchronous signal output | VCO COIL | 49 | Video detection |
| V OUT | 23 | Vertical signal output | VCO FIL | 50 | Video detection filtering |
| RAMP FIL | 24 | Forming and filtering for sawtooth wave | EXT AUDIO IN | 51 | External sound frequency input |
| VCC（H/D） | 25 | Power supply (horizontal oscillator/bus) | SIF OUT | 52 | Sound IF output |
| H APC FIL | 26 | Horizontal AFC filtering | SIF APC FIL | 53 | Sound frequency discrimination APC |
| H OUT | 27 | horizontal oscillating output | SIF IN | 54 | Sound IF input |

7.3 N304 TDA7056B   Function: Sound frequency power amplifying

| Symbol | PIN | Function description | Symbol | PIN | Function description |
| --- | --- | --- | --- | --- | --- |
| NC | 1 |  | OUT+ | 6 | Audio output at (+) termina |
| VP | 2 | Power supply | GND2 | 7 | GND2 |
| VI | 3 | Audio input | OUT- | 8 | Audio output at (-) terminal |
| GND1 | 4 | GND1 | NC | 9 |  |
| VC | 5 | Volume control |  |  |  |

7.4 N401 LA7840    Function: Vertical scan output

| Symbol | PIN | Function description | Symbol | PIN | Function description |
| --- | --- | --- | --- | --- | --- |
| GND | 1 |  | INV IN | 5 | Signal input |
| V OUT | 2 | Vertical deflection output | VCC2 | 6 | Power supply input |
| VCC2 | 3 | Power supply of output stage | PUMP UP | 7 | Pump power supply |
| NON INV IN | 4 | Input |  |  |  |

7.5 N601 MC44608P75    Function: Controlled oscillator of switching type power supply

| Symbol | PIN | Function description | Symbol | PIN | Function description |
| --- | --- | --- | --- | --- | --- |
| Demag | 1 | Switching transformer demagnetic switching transistor start protection | Driver | 5 | Output |
| ISENSE | 2 | V613 Sampling current for over-current protection | VCC | 6 | Power supply 6~13 V |
| Control input | 3 | Error voltage sampling input | NC | 7 |  |
| Gnd | 4 |  | Vi | 8 | Power supply for turn-on TV |

7.5.1 N601 MC44608P75

·MC44608 is used as a controlled oscillator in switching power supply, the oscillating output at its pin 5 is used as the input signal of the V613 power switching transistor. Thus, the output voltage of stabilized voltage power source can be controlled through controlling conduction period and duty cycle.

·The features of MC44608:

1. Few external elements/ more protections/ save energy – standby power consumption  3w.

2. Applicable range of input power is 90~220 V ac.

7.5.2 Overview of operation principle of the “green” power supply (refer to fig. 2)

·The operation principle of this power supply is same as general switching power supply. The output voltage is controlled through controlling of the conduction period and duty cycle of switching transistor. Only some new type elements such as controlled thyristorV642 MCR22-6 / Programmable zenan Diode V640 TL431LP are used in this power supply, besides MC44608 mentioned above.

·Programmable voltage-regulator TL431LP

Stabilized output voltage VKA= - [Vref(1+R1/R2)+Iref×R1]

The stabilized voltage is inverse proportion with sampled voltage.

input

VKA

Vref

Iref

IK

R1

R2

Cathode

(K)

Reference

(R)

Anode

(A)

·Stabilized process (assume +B increasing)

+B V640/Vref VKA Current of diode of photo-coupler Current of transistor of photo-coupler Voltage of N601/3 Conduction period of switching transistor of switching type power supply +B

·STANDBY and normal operation

Normal operation:

CPU N101/42 at high level  V604 saturated conduction  Thyristor V642G at low level  V642 cut-off  Power of photo-coupler N604/1 supplied by V614.

STANDBY operation:

CPU N101/42 at low level  V642 cut-off  Thyristor V642G at high level  V642 conducted  Power of photo-coupler N604/1 supplied by V642  Current of N604 increased largely  +B decreased to 17 V.

8. Voltage of IC and transistor

## N101 LC8632xxx(color bars)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 0 | 0 | 0 | 5 | / | 0/5 | / | / | 0 | 2 | 2.5 | 5 | 5 | 5 |
| PIN | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| Voltage V | 2.5 | 0 | 5 | 2.6 | 3 | 4.8 | 4.4 | 0 | 0 | 0 | 0 | / | 4.7 | 4.7 |
| PIN | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| Voltage V | / | / | 0 | 0 | 0.4 | 4.8 | / | / | / | 0 | 0 | / | / | 5 |

## N101 LC8632xxx(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 0 | 0 | 0 | 5 | / | 0 | / | / | 0 | 1.7 | 2.5 | 5 | 5 | 5 |
| PIN | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| Voltage V | 4.6 | 0 | 5 | 2.6 | 2.5 | 4.8 | 4.2 | 0 | 0 | 3.6 | 3.6 | / | 4.2 | 4.2 |
| PIN | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| Voltage V | / | / | 0 | 0 | 0.4 | 4.8 | / | / | / | 0 | 0 | / | / | 5 |

## N201  LA76814(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 2.3 | 2.3 | 2.5 | 1.9 | 2.9 | 2.9 | 0 | 5 | 2 | 2.3 | 4.7 | 4.7 | 4 | 0.8 |
| PIN | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| Voltage V | 0.8 | 1.1 | 0 | 8 | 2 | 2 | 2 | 0.4 | 2.2 | 2.7 | 5 | 2.6 | 0.7 | 1 |
| PIN | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| Voltage V | 1.6 | 0 | 0 | 2.7 | 0 | 2.6 | 2.1 | 3.9 | 2.4 | 2.9 | 3.6 | 2.5 | 0 | 2.6 |
| PIN | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 |  |  |
| Voltage V | 5 | 2.8 | 1.9 | 2.2 | 3.6 | 4 | 4 | 2.4 | 2.2 | 1.9 | 3.1 | 2.4 |  |  |

## N201  LA76814(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 2.3 | 2.3 | 0 | 5 | 2.9 | 2.9 | 0 | 5 | 2 | 4.6 | 4.3 | 4.3 | 3.7 | 1.1 |
| PIN | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| Voltage V | 1.1 | 3 | 1.8 | 7.6 | 1.2 | 1.2 | 2.6 | 0.2 | 2.4 | 2.6 | 5 | 2.6 | 0.7 | 1 |
| PIN | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| Voltage V | 1.6 | 0 | 0 | 2.7 | 0 | 0 | 1.3 | 3.4 | 0.3 | 2.9 | 2.3 | 2.5 | 0 | 2.6 |
| PIN | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 |  |  |
| Voltage V | 5 | 2.8 | 1.9 | 3.4 | 2.9 | 4 | 4 | 2.4 | 2.2 | 1.9 | 3.1 | 2.4 |  |  |

## N304  TDA7056(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 |  | 6 | 7 | 8 | 9 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | / | 14 | 2.4 | 0 | 0～ | 1.1 | 7 | 0 | 7 | / |  |  |  |  |

## N304  TDA7056(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 |  | 6 | 7 | 8 | 9 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | / | 14 | 2.4 | 0 | 0 |  | 7 | 0 | 7 | / |  |  |  |  |

## N401  LA7840(color bars)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 0 | 13 | 26 | 2.5 | 2.5 | 26 | 2 |  |  |  |  |  |  |  |

## N401  LA7840(no signals, blue background)

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 0 | 13 | 26 | 2.5 | 2.5 | 26 | 2 |  |  |  |  |  |  |  |

## N601  MC44608P75(color bars)

Normal operation

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 1 | 0 | 5 | 0 | 3.6 | 12.3 | 0 | 126 |

Standby operation

| PIN | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage V | 0 | 0．5 | Change | 0 | 0 | 8 | 0 | 131 |

Voltage of transistor (color bars)

|  |  | V103 |  |  |  | V107 |  |  |  | V108 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.3 | 4.8 | 0 | Voltage V | 0 | 16 | 0 | Voltage V | 16 | 0 | 15 |
|  |  | V114 |  |  |  | V203 |  |  |  | V302 |  |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.3 | 4.4 | 0 | Voltage V | 2.2 | 0 | 2.9 | Voltage V | 0.4 | 1.1 | 0 |
|  |  | V501 |  |  |  | V502 |  |  |  | V604 |  |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.16 | 19 | 0 | Voltage V | -0.1 | 115 | 0 | Voltage V | 0.7 | 0.2 | 0 |
|  |  | V613 |  |  |  | V640 |  |  |  | V642 |  |
| PIN | G | D | S | PIN | K | R | A | PIN | G | A | K |
| Voltage V | 3.6 | 140 | 0 | Voltage V | 6.5 | 2.5 | 0 | Voltage V | 7.1 | 111 | 7.8 |
|  | V901 | V911 | V921 |  |  | V701 |  |  |  |  |  |
| PIN | B | C | E | PIN | B | C | E |  |  |  |  |
| Voltage V | 1.7 | 120 | 1.2 | Voltage V | 2 | 11 | 1.4 |  |  |  |  |

Voltage of transistor (no signals, blue background)

|  |  | V103 |  |  |  | V107 |  |  |  | V108 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.3 | 4.8 | 0 | Voltage V | 0 | 16 | 0 | Voltage V | 16 | 0 | 15 |
|  |  | V114 |  |  |  | V203 |  |  |  | V302 |  |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.3 | 4.4 | 0 | Voltage V | 3.4 | 0 | 4.1 | Voltage V | 0.7 | 0 | 0 |
|  |  | V501 |  |  |  | V502 |  |  |  | V604 |  |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 0.16 | 19 | 0 | Voltage V | -0.1 | 115 | 0 | Voltage V | 0.7 | 0.2 | 0 |
|  |  | V613 |  |  |  | V640 |  |  |  | V642 |  |
| PIN | G | D | S | PIN | K | R | A | PIN | G | A | K |
| Voltage V | 3.6 | 140 | 0 | Voltage V | 6.5 | 2.5 | 0 | Voltage V | 7.1 | 111 | 7.8 |
|  |  | V911 | V921 |  |  | V701 |  |  |  | V901 |  |
| PIN | B | C | E | PIN | B | C | E | PIN | B | C | E |
| Voltage V | 1.3 | 185 | 1 | Voltage V | 2 | 11 | 1.4 | Voltage V | 2.6 | 108 | 2.3 |

9. Resistance to Earth of Some Circuits

(With multimeter 500 type×1K/red test pen connected to earth)

N101   （LC8632XXX）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / | 12 | 3.6K | 23 | 1.6K | 34 | 5K |
| 2 | / | 13 | 18K | 24 | 1.6K | 35 | / |
| 3 | / | 14 | 18K | 25 | 1.6K | 36 | / |
| 4 | 10K | 15 | 1.5K | 26 | / | 37 | / |
| 5 | / | 16 | 3.6K | 27 | 2.6K | 38 | 10K |
| 6 | 10K | 17 | 10K | 28 | 2.6K | 39 | 10K |
| 7 | / | 18 | 6K | 29 | / | 40 | / |
| 8 | / | 19 | 10K | 30 | / | 41 | / |
| 9 | 0 | 20 | 18K | 31 | 0 | 42 | 10K |
| 10 | 10K | 21 | 10K | 32 | 0 |  |  |
| 11 | 10K | 22 | 1.6K | 33 | 3K |  |  |

N102  （AT24C04）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | 0 | 5 | 3K | 7 | 0 |
| 2 | 0 | 4 | 0 | 6 | 3K | 8 | 3.6K |

N201 （LA76814）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.4K | 15 | 1.7K | 29 | 1.2K | 43 | 0.8K |
| 2 | 1.4K | 16 | 1.7K | 30 | 2K | 44 | 1.5K |
| 3 | 1.5K | 17 | 0.9K | 31 | / | 45 | 1.2K |
| 4 | 10K | 18 | 1K | 32 | 0.8K | 46 | 0.8K |
| 5 | 1.2K | 19 | 1.7K | 33 | 0 | 47 | 1.7K |
| 6 | 1.2K | 20 | 1.6K | 34 | 0 | 48 | 1.4K |
| 7 | 0 | 21 | 1.6K | 35 | 1.4K | 49 | 1.4K |
| 8 | 0.8K | 22 | 1.6K | 36 | 1.2K | 50 | 1.5K |
| 9 | 1.7K | 23 | 1.2K | 37 | 0.9K | 51 | 1.5K |
| 10 | 1.5K | 24 | 1.3K | 38 | 1.3K | 52 | 1.5K |
| 11 | 3K | 25 | 1.3K | 39 | 1.5K | 53 | 1.5K |
| 12 | 3K | 26 | 1.7K | 40 | 1.4K | 54 | 1.5K |
| 13 | 1.3K | 27 | 1.1K | 41 | 0 |  |  |
| 14 | 1.7K | 28 | 1.5K | 42 | 1.5K |  |  |

N304 （TDA7056B）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / | 3 | 100K | 5 | 10K | 7 | 0 |
| 2 | 10K | 4 | 0 | 6 | 10K | 8 | 10K |
|  |  |  |  |  |  | 9 | / |

N401（LA7840）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | 0.67K | 5 | 0.95K | 7 | 0.8K |
| 2 | 0.6K | 4 | 0.9K | 6 | 0.5K |  |  |

N601（MC44608P75）

| PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value | PIN | Resistance Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 90K | 3 | 4.4K | 5 | 1K | 7 | / |
| 2 | 3.4K | 4 | 0 | 6 | 36K* | 8 | 95K* |

*1. To measure with multimeter 500 type, the multiplying factor is [×10K], and the red test pen connected to earth.

The resistance to earth of load of each power supply (with multimeter 500 type×1K/red test pen connected to earth) are as follows:

VD621- =480		VD622- =500	VD623- =480

VD624- =460		VD621- =480

2. When the resistance of N601/PIN6 less than normal value, TV is in status of standby.

When the resistance of N601/PIN8 more than normal value, the switching power supply of TV is in status of stop.

10. Waveform Diagrams at Each Point

# Appendix 1  Explanation of Parts No. of elements

The designation of first digit:

| A | Complete set | S | Fastener |
| --- | --- | --- | --- |
| B | Component | T | Transformer |
| C | Capacitor | U |  |
| D | Diode | V | Variable resistor |
| E | Loudspeaker, earphone, microphone | W |  |
| F | Fuse | X | Filter, etc. |
| G | Mount of wiring | Y |  |
| H | Relay | R—X | Non-standard resistor |
| J | Conductor | C—X | Non-standard capacitor |
| K | Switch |  |  |
| L | Inductor |  |  |
| M | Accessories |  |  |
| N | IC |  |  |
| P | PCB |  |  |
| Q | Transistor |  |  |
| R | Resistor |  |  |

## Fixed resistor

R □ □ － － － □ □ □ □ □ □ □

1   2   3   4   5    6   7   8   9   10  11  12  13

4th,5th,6th digits : resistance value

13th digits: Manufacturer

10th digit: Shape of resistor

| A |  |
| --- | --- |
| B |  |
| C |  |
| S |  |
| D |  |
| E |  |
| F |  |
| G |  |
| H |  |
| J |  |
| K |  |
| L |  |
| M |  |
| N |  |
| P |  |
| Q | 3A |
| R |  |

2nd digit: Material

| D | Carbon film resistor |
| --- | --- |
| F | Fusible resistor |
| G | Glass grazed resistor |
| S | Metal oxide film resistor |
| C | Solid resistor |
| J | Metal film resistor |
| W | Wire resistor |
| Z | Bare copper wire |

7th digit: Error

| — | Non |
| --- | --- |
| A | +0.05% |
| B | +0.1% |
| C | +0.25% |
| D | +0.5% |
| F | +1% |
| G | +2% |
| J | +5% |
| K | +10% |
| M | +20% |
| N | +30% |
| P | +100%,-0% |
| Z | +80%,-20% |

9th digits: Characteristics

| N | Normal |
| --- | --- |
| A | Inflaming retarding |
| B | +50ppm/℃ |
| C | +100ppm/℃ |
| D | +200 ppm/℃ |

13th digit: Power

| — | Non |
| --- | --- |
| A | 1/8W |
| B | 1/6W |
| C | 1/4W |
| D | 1/2W |
| E | 1W |
| F | 2W |
| G | 3W |
| H | 5W |
| J | 6W |
| K | 7W |
| L | 8W |
| M | 9W |
| N | 10W |
| P | 15W |
| Q | 20W |
| R | 30W |
| S | 0.4W |
| T | 1/10W |

11th digit: Dimension of resistor body (mm)

| N | Bare copper wire |
| --- | --- |
| A | 3.5×1.8 |
| B | 6.5×2.3 |
| C | 9×3.5 |
| D | 12×4 |
| E | 15.5×6 |
| F | 23×9 |
| G |  |
| When 10th digit: D(Flake) |  |
| A | 1.6×0.8 |
| B | 2.0×1.25 |
| C | 3.2×1.6 |
| D | 3.2×2.6 |
| E | 5×2.5 |
| F | 6.4×3.2 |
| When 10th digit: F,G,H, J,K,L,M,N,P (wired) |  |

8th digit: Distance of lead(mm)

| — | Non |
| --- | --- |
| 1 | 5.0 |
| 2 | 7.5 |
| 3 | 10 |
| 4 | 12.5 |
| 5 | 15 |
| 6 | 17.5 |
| 7 | 20 |
| 8 | 22.5 |
| 9 | 25 |
| A | 27.5 |
| B | 30 |
| C | 32.5 |

12th digit: Type

| N | Bulk |  |
| --- | --- | --- |
| T | 52mm | Axial pop-up box |
| R | 26mm |  |
| B | 52mm | Axial disk form |
| C | 26mm |  |
| E | Flake disk form |  |
| F | Radial pop-up box |  |
| G | Radial disk form |  |

Note: When resistance value has 4 valid digits, do not use the first digit, shifting second 
~ fifth digits forward, and the third ~ sixth digits used as 4 valid digits.

Capacitor

C □ □ － － － □ □ □ □ □ □ □

1   2   3   4   5   6   7   8   9   10   11  12  13

4th,5th,6th digits: Capacitance value

13th digits: Manufacturer

2nd digit: Category

| C | Temperature compensated ceramic dielectric |
| --- | --- |
| K | High dielectric ceramic dielectric |
| B | Semiconductor ceramic dielectric |
| J | Monolithic |
| E | Aluminum-electrolytic |
| F | Polyester film(existed inductance) |
| D | Polyester film (no inductance) |
| G | Metallized polyester film |
| A | Metallized integrated layer polyester |
| N | Polypropylene file |
| M | Metallized Polypropylene film |
| H | Compound film |
| T | Tantalum-electrolytic |
| P | Non-polarity electrolytic |

3rd digit: Voltage

| A | 6.3V |
| --- | --- |
| B | 10V |
| C | 16V |
| D | 25V |
| E | 35V |
| F | 50V |
| G | 100V |
| H | 160V |
| J | 200V |
| K | 250V |
| L | 300V |
| M | 400V |
| N | 450V |
| P | 500V |
| R | 630V |
| W | 1KV |
| Z | 1.6KV |
| X | 2KV |
| Y | 63V |

10th digit: Shape

| A |  |
| --- | --- |
| J |  |
| B |  |
| C |  |
| D |  |
| E |  |
| F |  |
| G |  |
| H |  |
| K |  |
| L |  |
| M |  |
| N |  |

11th digit: Dimension of capacitor (mm)

| 10th digit Code | B | C D M N | E | F | Others |
| --- | --- | --- | --- | --- | --- |
|  | LxD | DxH | DxH | WxL | J.A.G.H |
| A | 3.5x1.8 | 5x11 | 22x30 | 1.0x0.5 | — |
| B | 6.5x2.3 | 6x11 | 22x40 | 1.6x0.8 |  |
| C | 9x3.5 | 8x11 | 26x40 | 2.0x1.25 |  |
| D | 12x4.5 | 10x12 | 26x50 | 3.2x1.6 |  |
| E |  | 10X16 | 30x60 |  |  |
| F |  | 10x20 | 34x60 |  |  |
| G |  | 12x20 | 35x35 |  |  |
| H |  | 12x25 | 22x35 |  |  |
| J |  | 16x25 | 35x50 |  |  |
| K |  | 16x30 | 35x45 |  |  |
| L |  | 18x40 | 22x45 |  |  |
| M |  | 20x30 | 30x40 |  |  |
| N |  | 20x35 | 35x30 |  |  |
| P |  | 22x40 | 30x50 |  |  |
| Q |  | 16x35 | 26x30 |  |  |
| R |  | 6x7 | 30x35 |  |  |
| S |  | 18x35 |  |  |  |
| T |  | 26x45 |  |  |  |
| V |  | 3x5 |  |  |  |
| X |  | 4x5 |  |  |  |

12th digit: Type

| N |  | Bulk |
| --- | --- | --- |
| T | 52 | Axial pop-up box |
| R | 26 |  |
| B | 52 | Axial disk form |
| C | 26 |  |
| E | Flake disk form |  |
| F | Radial pop-up box |  |
| G | Radial disk |  |

Digit of 7: Error

| A | +0.05% |
| --- | --- |
| B | +0.1% |
| C | +0.25% |
| D | +0.5% |
| F | +1% |
| G | +2% |
| J | +5% |
| K | +10% |
| M | +20% |
| P | +100-0% |
| Z | +80-20% |

8th digit: distance of lead (mm)

| — | Non |
| --- | --- |
| Y | 2.5 |
| 1 | 5 |
| 2 | 7.5 |
| 3 | 10 |
| 4 | 12.5 |
| 5 | 15 |
| 6 | 17.5 |
| 7 | 20 |
| 8 | 22.5 |
| 9 | 25 |
| A | 27.5 |
| B | 30 |
| C | 32.5 |

9th digits: Characteristics

| Code 2nd digit | A | B | C | E | F | G | S | P | X | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C |  |  | C? |  |  |  | SL | P | X | R |
| K |  | B | X7R | E | F |  |  |  |  | R |
| E.P | Normal 85 | 105℃ | Low leakage | High ripple | Long service life |  | 105℃high ripple |  |  |  |
| B |  |  |  |  | F |  |  |  |  | R |
| Others | — |  |  |  |  |  |  |  |  |  |

Non-standard resistor

R □ □ X □ □ □ □ □ □ □ □ □

1   2   3  4   5   6   7   8   9   10   11  12  13

2nd digit: Material

| D | Carbon film resistor |
| --- | --- |
| F | Fusible resistor |
| S | Metal oxide film resistor |
| C | Solid resistor |
| W | Wire resistor |
| J | Metal film resistor |
| Z | Bare copper wire |

3rd digit: Power

| A | 1/8W | G | 3W | N | 10W |
| --- | --- | --- | --- | --- | --- |
| B | 1/6W | H | 5W |  |  |
| C | 1/4W | J | 6W |  |  |
| D | 1/2W | K | 7W |  |  |
| E | 1W | L | 8W |  |  |
| F | 2W | M | 9W | — | Non |

5th,6th,7th digits: Resistance value

10th,11th digits: Serial no.

01~79(bare copper wire::80~99)

9th digit: Distance of lead (mm)

| 0 | — |
| --- | --- |
| 1 | 5.0 |
| 2 | 7.5 |
| 3 | 10 |
| 4 | 12.5 |
| 5 | 15 |
| 6 | 17.5 |
| 7 | 20 |
| 8 | 22.5 |
| 9 | 25 |
| A | 27.5 |
| B | 30 |

8th digit: Error

| A | +0.05% |
| --- | --- |
| B | +0.1% |
| C | +0.25% |
| D | +0.5% |
| F | +1% |
| G | +2% |
| J | +5% |
| LK | +10% |
| M | +20% |
| N | +30% |
| P | +100%,-0% |
| Z | +80%,-20% |
| — | Non |

13th digit: Manufacturer

12th digit: Type

| N | Bulk |  |
| --- | --- | --- |
| T | 52mm | Axial pop-up box |
| R | 26mm |  |
| B | 52mm | Axial disk form |
| C | 26mm |  |
| E | Flake disk form |  |
| F | Radial pop-up box |  |
| G | Radial disk form |  |

##### Non-standard Capacitor

C □ □ X □ □ □ □ □ □ □ □ □

1   2   3   4   5   6   7   8   9   10  11  12   13

3rd digit: Voltage

| A | 6.3V | K | 250V |
| --- | --- | --- | --- |
| B | 10V | L | 300V |
| C | 16V | M | 400V |
| D | 25V | N | AC250V |
| E | 35V | R | 630V |
| F | 50V | Z | 1.6KV |
| G | 100V | X | 2KV |
| H | 160V | Y | 63V |
| J | 200V |  |  |

12th digit: Type

| N |  | Bulk |
| --- | --- | --- |
| T | 52 | Axial pop-up box |
| R | 26 |  |
| B | 52 | Axial disk form |
| C | 26 |  |
| E | Flake disk form Radial pop-up box Radial disk form |  |
| F |  |  |
| G |  |  |

10th,11th digits: Serial no. 01~99

9th digit: Distance of lead (mm)

| 0 | — |
| --- | --- |
| 1 | 5 |
| 2 | 7.5 |
| 3 | 10 |
| 4 | 12.5 |
| 5 | 15 |
| 6 | 17.5 |
| 7 | 20 |
| 8 | 22.5 |
| 9 | 25 |
| A | 27.5 |
| B | 30 |
| C | 35 |
| Y | 2.5 |

13th digit: Manufacturer

5th,6th,7th digits: Capacitance value

| 2nd digit | 5th,6th digit | 7th,8th digit |
| --- | --- | --- |
| V | Minimum | Maximum |

2nd digit: Category

| C | Temperature compensated ceramic dielectric | G | Metallized polyester film |
| --- | --- | --- | --- |
| K | High dielectric ceramic dielectric | N | Polypropylene film |
| B | Semiconductor ceramic dielectric | M | Metallized Polypropylene film |
| J | Monolithic | H | Compound film |
| E | Aluminum-electrolytic | T | Tantalum-electrolytic |
| F | Polyester film (existed inductance) | P | Non-polarity electrolytic |
| D | Polyester film (no inductance) | V | Variable capacitor |

8th digit: Error

| A | +0.05% | J | +5% |
| --- | --- | --- | --- |
| B | +0.1% | K | +10% |
| C | +0.25% | M | +20% |
| D | +0.5% | P | +100-0% |
| F | +1% | Z | +80-20% |
| G | +2% |  |  |

Fuse

F □ □ － － － － □ □ □ □ — □

1   2   3   4   5   6   7   8    9   10  11  12  13

2nd digit: Type of fuse

| A | Normal |
| --- | --- |
| B | delay |
| C | Fast |
| D |  |

13th digit: Manufacturer

4th,5th,6th digits: Current (A)

7th digit: Multiplying factor

11th digit: Security number

| A | Australia |
| --- | --- |
| B | Argentina |
| C | China |
| D | Egypt |
| E | England |
| F | France |
| G | India |
| H | Hong Kong |
| J | Japan |
| M | Multi-mark |

3rd digit: Voltage

| code |  |  | Voltage |
| --- | --- | --- | --- |
| AC | DC | AC/DC |  |
| A | B | 1 | 250V |
| C | D | 2 | 127V |
| E | F | 3 |  |
| G | H | 4 |  |
| J | K | 5 |  |
| L | M | 6 |  |
| N | P | 7 |  |

9th ,10th digits: Material of tube, type and dimension

| Code | Dimension | Material of tube |
| --- | --- | --- |
| 01 | D3x20 | Glass |
| 02 | D5x20 | Glass |
| 03 |  | Plastic |

8th digit: Shape

| S |  |
| --- | --- |
| A |  |

# Switch

K □ □ □ □ □ □ □ □ □ — — □

1   2   3    4   5   6   7   8   9   10  11  12  13

6th digit:    Characteristics 1

| H | Horizontal |
| --- | --- |
| V | Vertical |

7th digit: Characteristics 2

| L | Locked |
| --- | --- |
| N | No lock |

13th digit: Manufacturer

8th ,9th ,10th digits: Serial no. 001—999

2nd ,3rd digits: Category

| PW | Push power switch | TD | Tetra-linkage push switch |
| --- | --- | --- | --- |
| PS | Slide power switch | SV | Lever switch |
| PA | Push button switch | LA | Slide switch |
| PC | Power select switch | HA | Single linkage switch |
| TA | Single linkage push switch(6x6mm) | HB | Bi-linkage switch |
| TB | Bi-linkage push switch (8x8mm) | KA | Keyboard switch |
| TC | Tri-linkage push switch(12x12mm) | RA | Rotary switch |

4th digit : Number of action points

| 1 | 1 position |
| --- | --- |
| 2 | 2 position |
| 3 | 3 position |
| 4 | 4 position |
| 5 | 5 position |
| 6 | 6 position |
| 7 | 7 position |

5th digit: Electrode number

| Code | Electrode number |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |

# Variable resistor

V □ □ □ □ □ □ □ □ □ □ □ □

1   2    3    4    5    6    7    8    9   10  11   12  13

6th ,7th ,8th digits: Resistance value

2nd digit: Material

| D | Carbon film |
| --- | --- |
| C | Glass graze |
| W | X wired |

3rd ,4th digits: Type category 1

| 3rd digit | 4th digit | Type | 3rd digit | 4th digit | Type |
| --- | --- | --- | --- | --- | --- |
| A | A | D6 horizontal | A | L | D14 horizontal |
| Rotary | B | D6 vertical | Rotary | M | D14 vertical |
|  | C | D8 horizontal |  | N | D16 horizontal |
|  | D | D8 vertical |  | P | D16 vertical |
|  | E | D9 horizontal |  | Q |  |
|  | F | D9 vertical |  |  |  |
|  | G | D10 horizontal |  | S | D19 horizontal |
|  | H | D10 vertical |  |  |  |
|  | J | D12 horizontal |  |  |  |
|  | K | D12 vertical |  |  |  |
| B | A |  |  |  |  |
| Slide | B |  |  |  |  |

10th ,11th ,12th digits: Serial no.  handle length, type, height, dimension of lead, center position

5th digit: Category 2

| A | Straight line type, without switch |
| --- | --- |
| B | Straight line type, with switch |
| C | Exponent type, without switch |
| D | Exponent type, with switch |
| E | Logarithm type, without switch |
| F | Logarithm type, with switch |
| G | S type, without switch |
| H | S type, with switch |

9th digit: Combination number

| A | 1 |
| --- | --- |
| B | 2 |
| C | 3 |
| D | 4 |
| E | 5 |
| F | 6 |

13th digit: Manufacturer

## Loudspeaker, Earphone, Microphone

E  □ □ □ □ □ □ □ □ □ — — □

1    2    3    4   5    6   7    8    9    10  11   12   13

2nd digit: Category

| S | Loudspeaker | M | Microphone |  |  |
| --- | --- | --- | --- | --- | --- |
| E | Earphone | P | Buzzer |  |  |

8th ,9th ,10th digits: Serial no.001~999

001—999

4th ,5th digits: Impedance (Ohm)

3rd digit: Sub-category

| 3rd digit 2nd digit | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
|  | External magnet | Internal magnet |  |  |  |
| S | Moving coil | Electromagnet |  |  |  |
| E | Moving coil | Electromagnet | Electret |  |  |
| M |  |  |  |  |  |
| P |  |  |  |  |  |

6th digit : Multiplying factor

13th digit: Manufacturer

7th digit: Type

| R | Circular |
| --- | --- |
| T | Elliptic |

Inductor

L □ □ □ □ □ □ □ — — — □ □

1   2    3    4    5   6    7    8    9   10   11  12   13

13th digit: Manufacturer

7th digit: Error

| J | +5% |
| --- | --- |
| K | +10% |
| M | +20% |
| “—” | When 2nd digit: C.F.E.M.G.T.N.P.Y |

2nd digit: Designation

| A | Vertical |
| --- | --- |
| B | Horizontal |
| C |  |
| D | Flake type |
| E | Magnetic bead |
| F | Horizontal linearity |
| G | Line filter |
| M | Degauss coil |
| T | Intermedium frequency coil |
| N | Antenna |
| P | Impedance convertor |
| Y | Deflection YOKE |

4th ,5th ,6th digits: Inductance value(H)

| 2nd digit | 4th ,5th ,6th digits |
| --- | --- |
| A.B.D | Inductance value |
| C.F.E.G.M | Serial no. 001~999 |
| T.N.P.Y |  |

12th digit: Type

| N |  | Bulk |
| --- | --- | --- |
| T | 52 | Axial pop-up box |
| R | 26 |  |
| B | 52 | Axial disk form |
| C | 26 |  |
| E | Flake disk form Radial pop-up box Radial disk form |  |
| F |  |  |
| G |  |  |

9th ,10th ,11th digits: “-”

| 3rd digit | 9th ,10th ,11th digits |  |
| --- | --- | --- |
| X | Serial no. |  |
| 2nd digit | 9th digit: Shape 10th ,11th digits: “-” |  |
| B |  |  |
|  | A |  |
|  | B |  |

8th digits: Distance of lead(mm)

| 0 | Special space |
| --- | --- |
| 1 | 5.0 |
| 2 | 7.5 |
| 3 | 10 |
| 4 | 12.5 |
| 5 | 15 |
| 6 | 17.5 |
| 7 | 20 |
| 8 | 22.5 |
| 9 | 25 |
| — | Non |
| 2nd digit: T(medium frequency) |  |
| C | With capacitor |
| N | Without capacitor |

3rd digit: Outline dimension (mm)

| 2nd digit Code | A | B | D | E | T | N |
| --- | --- | --- | --- | --- | --- | --- |
|  | D | D×L | L×W | D×L×d | L×W | type |
| A | 6 | 2x4 | 3.2x2.5 | 3x6x1 | 4x4 | Tie |
| B | 8 | 3x5 | 2x1.25 | 3.5x9x0.6 | 5x5 |  |
| C | 10 | 3x7 |  | 3x7x1 | 7x7 |  |
| D | 18 | 4x10 |  | 3.5x5x1.3 | 10x10 |  |
| E | 12.5 |  |  | 6x6x2 |  |  |
| F |  |  |  | 5x10x1.8 |  |  |
| G |  |  |  | 3.5*4.5*0.8 |  |  |
| H |  |  |  | 5x6x2 |  |  |
| J |  |  |  | 10x6x3 |  |  |
| X | Non-standard |  |  | Specific |  |  |
| “—” | 2nd digit: C.F.G.M.P.Y |  |  |  |  |  |

Transformer

T □ — □ □ □ □ — — — — □ □

1   2    3   4    5    6   7    8    9   10   11  12   13

4th ,5th ,6th ,7th digits: Serial no. 0001~9999

Second digit: Dielectric material

| B | Iron core (for power supply) |
| --- | --- |
| M | Magnetic core (for power supply) |
| F | FBT |
| D | Horizontal drive |

12th digit: Changed serial no. 0~9

13th digit: Manufacturer

Integrated circuit (IC)

N □ □ □ □ □ □ □ □ □ □ □ □

1   2   3    4    5    6   7    8    9   10   11  12   13

12th digit: Type

| N | Bulk |
| --- | --- |
| E | Flake disk form |
| F | Radial pop-up box |
| G | Radial disk form |

13th digit: Release code A…                common type indicated with “_”.

Second 2~11th digits: Type indicated directly, blank indicated with “_”

Transistor and MOS transistor

Q □ □ □ □ □ □ □ — □ □ □ □

1   2    3   4    5    6    7   8   9    10   11   12  13

11th digit: Integral form

| A | 2 |
| --- | --- |
| B | 2 |
| C |  |
| D |  |
| E、F、G、 H、J、K |  |
| N | Non |

Front

Front

1,3

10th digit: Amplified factor indicated directly, for example: Toshiba O, Y, GR.  Code: O, Y, G.  “_” indicates non.

1.3

1.3

2

Second ~8th digit: Type indicated directly, and blank indicated with “_”.

12th digit: Type

| N | Bulk |
| --- | --- |
| E | Flake disk form |
| F | Radial pop-up box |
| G | Radial disk form |

Front

13th digit: Manufacturer (domestic)

# Semiconductor

D □ □ □ □ □ □ □ □ □ □ □ □

1   2    3    4   5    6    7    8   9   10   11   12  13

Second digit: Category

| R | Diode |
| --- | --- |
| Z | Zenar diode |
| L | Light-emitting diode |
| H | Thermal element (PTC) |
| N | Thermal element (NTC) |
| V | Pressure-sensing element |
| G | Light-sensing element |

3rd ~11th digits: Type indicated directly, blank indicated with “_” (If the shape of final products have special requirement, which can not be indicated in the type, then it can be indicated by serial no. of 10th and 11th digits)

2nd digit: H, N, V

| 3rd digit |  | 4th ,5th , 6th digits | 7th digit | 8th ,9th ,10th digits | 11th digit |
| --- | --- | --- | --- | --- | --- |
| Internal construction |  | Resistance value in normal condition | Error | Serial no. | “—” |
| 1 | 2 |  |  |  |  |
| Single element | Bi-element |  |  |  |  |

13th digit: Manufacturer

2nd digit: L

| 3rd digit: Category |  | 4th digit: Dimension |  |  | 5th digit |  | 6th,7th,8th digits | 9th ,10th , 11th digits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Designation | Code | Circle | Rectangle | Code | Color | Serial no. | “—” |
| A | Circle(without color) | 2 | D2 | 5x1 | 2 | Red | 001-999 |  |
| B | Circle (transparent) | 3 | D3 | 5x2 | 3 | Orange |  |  |
| C | Circle (scatter) | 4 | D4 |  | 4 | Yellow |  |  |
| D | Circle (bi-color) | 5 | D5 |  | 5 | Green |  |  |
| E | Rectangle |  |  |  | A | Orange, yellow-green |  |  |
| F | Rectangle (bi-color) |  |  |  | B | Red, green |  |  |

2nd digit “V”: 3rd ~7th categories, serial no. 8-10

12th digit: Type

| N |  | Bulk |
| --- | --- | --- |
| T | 52 | Axial pop-up box |
| R | 26 |  |
| B | 52 | Axial disk form |
| C | 26 |  |
| E |  | Flake disk form |
| F |  | Radial pop-up box |
| G |  | Radial disk form |

SAW Filter, Filter, Trapper, discriminator, Delay-line, Resonator

X □ □ □ □ □ — — — — — □ □

1   2   3    4    5   6    7    8    9   10   11  12   13

Sound digit: Category

| F | Filter |
| --- | --- |
| T | Trapper |
| D | Discriminator |
| B | Brightness delay-line |
| C | Color delay-line |
| R | Ceramic resonator |
| X | Crystal resonator |

3rd digit: Sub-category

| 2nd digit Code | F | T | D | B.C.R.X |
| --- | --- | --- | --- | --- |
| S | SAW Filter |  |  |  |
| C | Ceramic | Ceramic | Ceramic |  |
| L | LC type | LC type | LC type |  |
| “—” |  |  |  | “—” |

4th ,5th ,6th digits: Serial no. 001~999

13th digit: Manufacturer

12th digit: Type

| N | Bulk |
| --- | --- |
| E | Flake disk form |
| F | Radial pop-up box |
| G | Radial disk form |

# PCB

P □ □ □ □ □ □ □ — — —  □ □

1   2    3   4   5    6    7    8    9   10   11   12   13

2nd digit: Category

| A | Main board | T | Tuner board |
| --- | --- | --- | --- |
| B | CRT board | X | Hybrid board (mixed) |
| C | Power supply board | Y | Other sub board |
| D | MF amplifier board | P | PIP board |
| E | Sound frequency board |  |  |
| F | Keyboard board |  |  |
| G | Interface board |  |  |

4th ,5th ,6th ,7th digits: Serial no. 0001~9999

8th digit: Serial no. of hybrid board in same category: “_”, for in different category: A, B…

3rd digit: Hybrid board quantity

| 3rd digit of Code | Quantity of hybrid board |
| --- | --- |
| —— | Non |
| 2...9 | 2...9 |
| 10...20 | A...K |

12th digit: Change serial no. 1~9

13th digit: Manufacturer

Socket

G □ □ □ □ □ □ □ — — — — □

1   2    3    4   5    6    7   8    9   10   11  12   13

2nd, 3rd digits: Category 1

| 2nd digit | 3rd digit | Designation |
| --- | --- | --- |
| A | A | TJC1 big size |
|  | B | TJC2 medium size |
|  | C | TJC3 (small size) XH |
|  | D | TJC3(small size)EH |
|  | E | MKS STOCKO base |
|  | F | MKS STOCKO PLUG |
|  | J | B TO B STOCKO |
|  | Z | TJC18A |
|  | Y | TJC18Y (base) |
|  | G | IC base (bi-row) |
|  | H | IC base (single row) |
|  | K | VH base |
| B | A | CRT base |
|  | B | RCA base |
|  | C | SVHS base |
|  | D | 3.5mmJACK base |
|  | E | 2.5mmJACK base |
|  | F | 6.35mm JACK base |
|  | G | DIN base |
|  | H | 3-pole power base |
| C | A | wire clip |

5th digit: Number of contacts

| Code | Number of contacts |  | Code | Number of contacts |
| --- | --- | --- | --- | --- |
| 1 | 1 |  | 8 | 8 |
| 2 | 2 |  | 9 | 9 |
| 3 | 3 |  | A | 10 |
| 4 | 4 |  | B | 11 |
| 5 | 5 |  | C | 12 |
| 6 | 6 |  | J | 18 |
| 7 | 7 |  |  |  |
| 2nd ,3rd digits: AG, AH |  |  | 2nd ,3rd digits: AB |  |
| Code |  | Number pins | Code | Number pins |
| A |  | 52 | S | Small diameter |
| B |  | 40 | L | Big diameter |
| C |  | 30 |  |  |
| E |  | 42 |  |  |
| F |  | 8 |  |  |
| D |  | 64 |  |  |

4th digit: Category 2

| V |  | Vertical |
| --- | --- | --- |
| H |  | Horizontal |
| A |  | Vertical with switch |
| B |  | Horizontal with switch |
| 2nd ,3rd digits | 4th digit | Space(mm) |
| AG | A | 2.54 |
| AH | B | 1.78 |

13th digit: Manufacturer

6th ,7th ,8th digits: Serial no. 001~999 (2nd ,3rd digit: BB)

| Code | 6th digit: | 7th ,8th digit: |
| --- | --- | --- |
|  | Color | Serial no. |
| 2 | Red | 01---99 |
| 4 | Yellow |  |
| 9 | White |  |
| ─ | Mix |  |

# Conductor material

J □ □ □□ □ □ □ □ □ □ — □

1  2   3   4   5   6   7  8   9   10  11  12  13

8th digit: Plug type

| Code | Type | Security code (2nd digit: P) |
| --- | --- | --- |
| A | TJC1(Big size) | Australia |
| B | TJC2(medium size) | Argentina |
| C | TJC3(small size)-XH | China CCEE |
| D | TJC3(small size)-EH | Egypt |
| E | TJC3-XH—D2 | England |
| F | TJC3-EH—D2 | France |
| G | IEC—IEC | India |
| H | IEC—RCA | Hong Kong |
| J | D2—D2 | Japan |
| L | MKF STOCKO | Germany VDE |
| M | VH |  |
| K | XH-XH |  |
| V |  | VDE |
| U |  | UL |
| X | Unknown |  |
| — | Non |  |

9th ,10th ,11th digits: Serial no. 001~999

13th digit: Manufacturer

4th ,5th ,6th digits: Length (mm)  (4th ,5th digits: Valid figures  6th digit: Multiplying factor)

2nd digit: E  4th ,5th ,6th digits: “_”

3rd digit: Number of wire cores

| 1 | 1 core |
| --- | --- |
| 2 | 2 cores |
| 3 | 3 cores |
| 4 | 4 cores |
| 5 | 5 cores |
| 6 | 6 cores |
| 7 | 7 cores |
| 8 | 8 cores |
| 9 | 9 cores |
| A | 10 cores |
| B | 11 cores |
| F | 15 cores |

7th digit: “_”

2nd ,3rd digits: S1(divergent wire)

| Code | Color |
| --- | --- |
| 1 | Brown |
| 2 | Red |
| 3 | Orange |
| 4 | Yellow |
| 5 | Green |
| 6 | Blue |
| 7 | Violet |
| 8 | Gray |
| 9 | White |
| 0 | Black |
| — | Others |

2nd digit: category

| P | Power supply wire |
| --- | --- |
| E | GND conductor |
| W | Hybrid wire |
| H | High insulation wire |
| B | Shielding wire |
| D | Bi-layer insulation |
| C | Coaxial cable (IEC) |
| S | Divergent wire |
| Z | Copper wire(tinning) |

## Bolt, Screw

S □ □ □ □ □ □ □ □ □ □ — □

1   2    3    4   5    6   7    8    9   10   11  12   13

13th digit: Manufacturer

2nd digit: Category 1

| A | Bolt GB | M | Notch head tap screw GB |
| --- | --- | --- | --- |
| B | Bolt SJ | N | Notch head tap screw SJ |
| C | Screw GB | P | Flat head tap screw GB |
| D | Screw SJ | Q | Flat head tap screw SJ |
| E | Combined Screw GB | R | Combined sharp head tap screw GB |
| F | Combined Screw SJ | S | Combined sharp head tap screw SJ |
| G |  | T | Combined notch head tap screw GB |
| H |  | U | Combined notch head tap screw SJ |
| J |  | Y | Non-standard tap screw |
| K | Sharp head tap screw GB |  |  |
| L | Sharp head tap screw SJ | X | Nut and washer |

3rd digit: Category 2

| A | Cross pan head |
| --- | --- |
| B | Cross pan head with washer |
| C | Cross sunk head |
| D | Cross hex head |
| E | Cross hex head with washer |
| F | Socket hex head |
| G | Cross flat head |

4th digit: Category 3 (suitable for components, others indicated with “_”)

| A | With flat washer |
| --- | --- |
| B | With spring washer |
| C | With flat and spring washer |
| D | With quincunx washer |

7th ,8th ,9th digits: Length (mm)

| L<10mm | L>=10mm |
| --- | --- |
| Decimal: indication with R | No indication of decimal |

Example:L=6mm  6R0  L= 350mm 350

5th ,6th digits: Diameter (mm)

1. D<10mm: direct indication (no indication for decimals)

2. D10mm: tens digit indicated with code

| A | 10 | D | 40 | G | 70 |
| --- | --- | --- | --- | --- | --- |
| B | 20 | E | 50 | H | 80 |
| C | 30 | F | 60 | J | 90 |

Example: D=15mm     A5

D=3.0mm     30

D=20mm     B0

10th digit: Material

| A | B | C | U | E | F | G | N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Al |  | Cu | Stainless steel |  | Normal steel |  | Nylon |

11th digit: Coating material

| A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| Ni | Cr | Cu | Zinc black | Zinc yellow | Zinc white |

# Nut, Washer

S  X □ □ □ □ □ — — □ □ — □

1    2    3   4    5    6   7    8    9   10   11   12  13

13th digit: Manufacturer

3rd digit:

NNutWWasher

Washer: 4th digit: Category 1

1GB2SJ3Non-standard

5th digit: Category 2

ANormal hexHCombined with flat washerBFine thread hexJCombined with spring washerCSquareKCombined with flat and spring washerDWingLCombined with quincunx washerEThin capMCombined with bowl-form washerFThick capNGHex with washer

Nut: 4th digit: Category 1

1GB2SJ3Non –standard

5th digit: Category 2

AStandard flat washerGExternal tooth flat washerBSmall size flat washerHBoth internal and external tooth flat washerCBig size flat washerJConical external tooth washerDCircular spring washerLQuincunx washerEFlat spring washerMBowl-form washerFInternal tooth flat

washer

6th , 7th digits: Diameter (mm)

1. D<10mm: direct indication (no indication for decimals)

2. D10mm: tens digit indicated with code

| A | 10 | D | 40 | G | 70 |
| --- | --- | --- | --- | --- | --- |
| B | 20 | E | 50 | H | 80 |
| C | 30 | F | 60 | J | 90 |

Example:D=15mm     A5

11th digit: Coating material (nut, washer)

| A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| Ni | Cr | Cu | Zinc black | Zinc yellow | Zinc white |

10th digit: Material (nut, washer)

| A | B | C | U | E | F | G | N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Al |  | Cu | Stainless steel |  | Normal steel |  | Nylon |

Component

B □ □ □ □ □ □ □ — — — □ □

1    2   3    4   5    6   7    8   9   10   11   12   13

3rd digit: Category

| 2nd digit: X(chassis) | A | Main board unit | E |  | Graph-words unit | 2nd digit: C (housing product) | A | 9” | J | 21” |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B | Video unit | F |  | PIP |  | B | 12” | K | 22” |
|  | C | Interface unit | G |  |  |  | C | 14” | L | 24” |
|  | D | Keyboard unit | H |  | Auxiliary unit (synthesized) |  | D | 16” | M | 28” |
| 2nd digit: U(tube) | A | 14” | H | 22” |  |  | E | 17” | N | 29” |
|  | B | 16” | J | 24” |  |  | F | 18” | P | 33” |
|  | C | 17” | K | 25” |  |  | G | 19” | Q | 34” |
|  | D | 18” | L | 28” |  |  | H | 20” |  |  |
|  | E | 19” | M | 29” |  | 2nd digit: T (emitter) | 3rd digit: Indication of “_” |  |  |  |
|  | F | 20” | N | 33” |  |  |  |  |  |  |
|  | G | 21” | P | 34” |  |  |  |  |  |  |

2nd digit: Category

| X | Inner-works |
| --- | --- |
| C | Housing |
| U | CRT |
| T | Remote emitter |

4th,5th,6th,7th digits: serial no.    0001—9999

8th digit: Changed serial no. A

12th,13th digits: Manufacturer

- Tuner：BXAT□□□□□———□

6th,7th,8th digits: Serial no.

13th digit: Manufactory

5th digit: Category

| N | Normal | S | Satellite bands |
| --- | --- | --- | --- |
| A | Supplement300MHZ | K | Satellite Ku band |
| B | Supplement470MHZ |  |  |

9th digit:

| ─ | Voltage synthesis |
| --- | --- |
| F | Frequency synthesis |

- Remote receiver  BXAR□□□□————□

13th digit: Manufacturer

5th,6th,7th,8th digits:

0001--9999

Complete set

A □ □ □ □ □ □ □ — — — — □

1   2    3   4    5    6    7    8   9   10   11   12   13

2nd,3rd digits: Category

| 90 | TV set |
| --- | --- |
| 20 | Audio equipment |
| 30 | Monitor |

4th digit: Serial no. of trademark

| M | SANWAN |
| --- | --- |
| H | HUANYU |

5th,6th,7th digits: Product serial no.

001—999

13th digit: Manufacturer

8th digit: Improved serial no.

0—9

Thick film component

B  -  □ □ □ □ □ □ □ □ □ □ □

1    2     3    4   5    6   7    8    9   10   11  12   13

| 第3位 | 第9 位 |
| --- | --- |
| A.B | 误差 |
| C.D.E.F | — |

AB1 2   3 4   5 6  2n-1 2n CDEF    Others“—”

1 2  3 4 5...n+1

1 2     3   n  n+1 n+2

2R 2R  2R  2R  2R

R  R   R

4th digit: Net type (3rd digit:ABC)

5th digit: Number of elements in net

| 3rd digit: A,B resistor, capacitor | Code | Quantity |
| --- | --- | --- |
|  | 1...9.A.. | 1...9.10... |
| 3rd digit: C RC component | Code | Quantity |
|  | 1...9.A.. | 1...9.10... |
| 3rd digit: D,E,F active element | “—” |  |

3rd digit: Category

| A | Pure resistance net |
| --- | --- |
| B | Pure capacitance net |
| C | RC net |
| D | Active net (power supply) |
| E | Active net (high power) |
| F | Active net (small signals) |

6th,7th,8th digits:

9th digit:

13th digit: Manufactory

11th digit: Lead distance

A:2.54mm

B:1.78mm

12th digit: Type

F: Radial pop-up box

N Bulk

10th digit: Material characteristics

Accessories

M  □ □ □ □ □ □ □ — — — □ □

1    2    3    4   5    6    7   8    9   10   11   12   13

3rd digit: Category2

| 2nd digit 3rd digit | A | B | C | E | F | G | H | J | K | L | M | N | T | P |  | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | PVC label | Support | Rubber block | X | Circuit diagram | Plastic bag | Tension spring | Set cushion | Complete set | SANWAN | Ring form | Hollow | R6S | Terminal plate |  | Sheet mica |
| 1 | Silicon gel | Radiator | Rubber ring |  | Instruction | Plastic nail | Press spring | Sponge cushion | Parts | NANKONG |  |  |  |  |  |  |
| 2 | Adhesive tape | Grid cover | Rubber muscle |  | Maintenance Card |  |  |  | Turnover sheet |  |  |  |  |  |  |  |
| 3 | Self-adhesive paper | Metal spacer | Conduct heat film |  | Conformity certificate | Plastic tube clip |  |  |  |  |  |  |  |  |  |  |
| 4 | Self-adhesive non-woven | Metal bar |  |  |  |  |  |  |  | Domestic trademark |  |  |  |  |  |  |
| 5 | Damped block | Metal hook |  |  |  |  |  |  |  | NEC |  |  |  |  |  |  |
| 6 | Bar code paper | Shield cover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Label for product no. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2nd digit: Category1

| A | Adhesive | S |  | Discharger |
| --- | --- | --- | --- | --- |
| B | Metal support, flake, hook | T |  | Battery |
| C | Rubber | U |  |  |
| E | Cabinet lock | V |  |  |
| F | Sheet | W |  |  |
| G | Plastic | X |  |  |
| H | Spring | Y |  |  |
| J | Foam cushion | 1 |  |  |
| K | Carton | 2 |  |  |
| L | Trademark | 3 |  |  |
| M | Magnetic material | 4 |  |  |
| N | Rivet | 5 |  |  |
| P | Terminal plate | 6 |  |  |
| Q |  | 7 |  |  |
| R |  | 8 |  |  |
|  |  | 9 |  | Unknown |
| D: Injection molding |  |  |  |  |
| 3rd digit: Category 2(Screen size code) |  |  |  |  |
| A | 14” | J | 24” |  |
| B | 16” | K | 25” |  |
| C | 17” | L | 28” |  |
| D | 18” | M | 29” |  |
| E | 19” | N | 33” |  |
| F | 20” | P | 34” |  |
| G | 21” |  |  |  |
| H | 22” |  |  |  |

5th,6th digits: Cabinet parts serial no. 01~99

7th digit: Material

8th digit: Changed serial no. 1~9

| A | ABS |
| --- | --- |
| B | 聚丙烯 |
| P | HIPS |
| G | 透明塑料 |
| N | 尼龙 |

12th,13th digits: Manufacturer

4th digit: Category3 (code of housing serial no.)

3th digit

4th digitA(14”)F（20”）G(21”)K(25”)M(29”)P(34’)AEK-AEK1飞跃1陆氏1BNEC1EK2金丰1 NEC29’CNEC2SingleEK3华夏2598DNEC3TwoEPANDA1401NEC5FHAIFENGGHJEK94th digit: X5th,6th,7th,8th digits: Serial no.

Appendix 2 Difference of CRT

| Tube type | PHILIPS A51EFK155X01 | SEG-HITACHI A51JFC82X13（C） |
| --- | --- | --- |
| Main power voltage | 117V | 110V |
| R951 | RFE3R9J5ACDNF | RFF1R2J5ACDNF |
| C522 | CKX681K2RL-ND | CKX471K2RL-ND |
| C514 | CMK364J5-K-NP/S | CMK394J5-K-NP/S |
| L501 | LF-037-2---NJ | LF-015-2---NJ |
| R431 | RSE821J4ACCND | RDD102J4NBCND |
| R639 | RDD513J4NBCND | RDD473J4NBCND |
| T601 | TM-0047----1L | TM-0047----0L |